# Conclusion

In our reliability model of the Safety Amplifier we initially assumed that the component failures were statistically independent and that all system failures were caused by individual component failures. Calculated unavailabilities for all cut sets in this model are shown to be rather small, typically less than 10-10, so that it is reasonable to expect that some other failures, possibly caused by external means, would dominate the unavailabilities and thus have to be included in the analysis. These failures are called common cause failures (CCF), since they can affect more than one component at the time. All safety systems, designed as redundant systems, utilize the idea of reducing the system failures caused by internal means by using parallel connections and the notion of suitably chosen monitoring and repair. As a consequence, the majority of the safety system failures are in fact common cause failures. These failures, however, may be so rare that other methods, in addition to the classical statistical inference analysis, may have to be used. These methods include Bayesian analysis that may provide results when censoring insufficiency, i.e. a lack of the statistical data, is in effect. It is sensible when considering CCF to include only those events that may happen during the system’s operational lifetime, and among them only those that can happen without any prior indication, since the opposite would allow a well-timed preventive counteraction. In that sense, for example, an airplane crash is an appropriate chosen CCF, while a flood is not, even if the former might be more probable to happen. In our investigation CCFs are not systematically portrayed. Most of the external CCFs, in fact, will also affect the rest of the MNR system, so that they deserve to be analyzed independently, outside the scope of the current analysis. However, in order to compare the magnitude of these effects to the current results, a CCF model is included in the analysis in the form of the beta factor model with arbitrarily chosen reliability parameters that are likely to be close to some real parameters. It was shown that the CCFs form the cut sets with the highest importance parameters and that they dramatically increase both the unavailability and the failure frequency of the Safety Amplifier. By excluding common cause failures, we showed that the Safety Amplifier is inherently safe with resulting unavailability of 1.39x10-6 and failure frequency of 6.6x10-3y-1 given the input scram signal came from uncompensated chambers, and with resulting unavailability of 8.06x10-6 and failure frequency of 4.17x10-3y-1 given the input scram signal came from the Log-N Amplifier. The component that was found to be most likely to cause the Safety Amplifier failure is the resistor R49. This should not come as a surprise, since R49 does not have a redundant counterpart and therefore forms a bottleneck for the fast scram.

The program used for reliability calculation was FaultTree+. This program was found to be suitable for the basic reliability modeling of the Safety Amplifier. However, FaultTree+ like many other available programs is more suitable for reliability than for safety analysis. Although the common cause failures are supported, different paths in the reliability graph used for the tests can not be accounted for.

Possible extensions of the current work include the following:

- gathering more relevant statistical data, including internal data from the forty years of the MNR operating experience. In particular, these data should include common cause failures. Most common cause failures are specific to the particular equipment setup, and their probabilities cannot be reliably obtained using the data from external sources.

- applying Bayesian analysis for rare events. High uncertainties related to the probabilities of rare events may, in certain cases, be reduced by assigning appropriate prior probability densities to the events based on the knowledge acquired by some earlier experience. This technique combines the (posterior) evidence with the (prior) knowledge, and is in the heart of the variety of methods that utilize “plausible reasoning” for automation of the process that is as close to the process of human thinking as we can imagine.

- developing suitable models that can be applied to multi-input systems. As explained earlier, unavailabilities of individual inputs cannot be linearly combined to yield the “resulting unavailability” of the system because of the strong dependencies between the inputs, both due to the CCF and to the shared hardware. For purposes of the safety analysis, it is desirable to divide the hardware in such a way that the failure covariance matrix in the given representation always has a diagonal form. Some techniques from tensor analysis can also be applied.

- developing suitable models that can handle different levels of supervision. This part is important because it affects not just the “post-mortem” reliability analysis, but also the designing process itself. Supervision essentially deals with non-coherent systems. Supervision includes state annunciators, or sensors, and involves the process called decision-making. As a result of the decision-making process, the topology of the reliability graph can change, e.g. switches can be formed, etc. These models can also be applied to human factors.

- developing suitable models that can account for the human errors. A comprehensive and quantitative analysis of the human errors is nowadays still missing. The hardware reliability, unlike the human reliability, in some cases can be very high. On the other hand, humans estimate risk poorly, and cause the vast majority of all kinds of failures.

- collecting the relevant data and performing more accurate CCF analysis. Beside the CCF data, the analysis of the CCF data itself could be improved. Taking into account the important role that the common cause failures play in highly reliable systems, they deserve to be considered with more thoroughness. Every well-designed safety system is more likely to fail due to a common cause than due to a random combination of isolated failures.