Chapter 4

# Common Cause Failures

Common cause failures (CCF) are those failure events that can cause a failure of several components within a system simultaneously. CCF represent the vast majority of failures in all safety systems. This is because a safety system is designed to reduce individual component failures by several orders of magnitude, most commonly by utilizing redundancy, while some common failure events remain intact. At this point the probability of failure is already so low that we encounter another problem characteristic to common cause failures. Namely, their statistics are usually very poor. Common cause failures may be divided in several categories in several ways:

- By nature of origin (manmade, or decision related failures, and hardware, or random failures)

- By place of origin (external failures, internal failures)

- By nature of statistical correlation (coherent, cross-correlated)

External common cause failures, such as a fire or an earthquake, may destroy the whole Safety Amplifier and even cause a significant collateral damage. A spare Safety Amplifier is kept as a part of a logistics, or a maintenance policy. It should be kept at a separate place, possibly in the Reactor Building, but outside the control room. Internal CCF’s are caused by environmental conditions such as vibration, moisture, or rapid changes in temperature. These conditions must be kept under control at all times, even when the reactor is shut down. The training of the control room staff and regular, leveled supervision is a sine qua non of the safety practice. As experience shows, manmade errors are the leading cause of all accidents in a variety of human activities, including nuclear engineering. Finally, CCF analysis may show in which cases a defense in depth, in spite of the higher cost, may be more appropriate strategy employed against failures than the simple usage of redundant systems.

## 4.1.  Power Supply

All active elements including the relays in the Safety Amp are normally energized. This condition corresponds to a no-scram signal (s=0). If the scram signal (s=1) reaches the input of a unit, the unit’s relay current would drop and the relay would open, transferring the scram signal (s=1) from the input to the output. If the two operating modes, namely s=0 and s=1, were inverted, the power supply failure would become the most significant common cause failure (CCF) of the system. Namely, in case of power supply failure, all active components would be affected simultaneously, and the reliability enhancement we wanted to achieve by redundancies would be lost. By identifying the no-scram signal with energized state, this was averted. Moreover, the unavailability of the power supply has become of no concern to us at all, since whenever the power supply fails the system will be shut down. From that point of view, the failure of the power supply is just another scram signal, and for that reason the power supply is not even considered as a part of the Safety Amp. In addition to that, all events that happen before the signal enter the power supply, and that, except for the path through the power supply, have no redundant signal propagation paths, are also eliminated from the fault tree. This includes some types of hydroelectric fluctuations and all hydroelectric failures. This is an important fact, since the frequencies of these events essentially cannot be reduced by enhancement in power supply design. This solution also eliminates a need for a standby power supply unit. Normally, a power supply is designed to reduce any abrupt or severe changes in the output voltage. Nevertheless, some CCFs can be caused by the loss of trimming in the power supply as a result of a damaged unit. This may result in some relays being ‘welded’ in the closed position, and therefore in the loss of redundancy functions of the Safety Amplifier.

## Operator Error

Human operators may make the same type of mistake in how they operate switches, maintain systems, approach problems, etc. Therefore the same human error may lead to loss of multiple functions in the system. For example, connecting incorrectly the cable to UIC1 is likely to result in similar operator’s error at UIC2 since the same manual procedure is taken in both cases. Or, warning systems may be disabled because they create spurious warnings, which may happen if a radioactive sample is being carried through the MNR building. Operator errors may include incorrect setup, monitoring, maintenance, and improperly conducted checkouts of the Safety Amplifier. Quantifying human error probabilities for purposes of probabilistic safety analysis is related to immense difficulties. Therefore, human errors will not be included in the current report. We cannot include human errors in the Safety Amplifier fault tree simply because the leaves of the tree, or the basic events that are causing human errors, are not known.

## Failure on Demand

Parallel systems are particularly sensitive to a failure on demand. If we consider the (non-demand) reliability of two components in parallel for a short period of time after t=0

(4.1)

we may conclude that, unlike for the single component case, at proximity of time t0 the

parallel system virtually cannot fail. That is because

(4.2)

Let us assume that

- we can effectively and instantaneously determine if any one of the two components has failed, and

- the system is not allowed to run under repair.

Both of these assumptions may be applied for some of our redundancy subsystems, such as the FSP subsystem in the Safety Amp. This would lead us to a conclusion that the FSP subsystem might never fail. Namely, because of the instantaneous repair the system is virtually always at the beginning of the reliability life-cycle where all four units are working, and where according to above formula the subsystem cannot fail. As a consequence, the common mode failure would become the only failure type that needs to be included in the analysis. However, due to the functioning nature of the most failure prone components in the Safety Amp, the relays, one additional failure mode is to be considered. This is a failure on demand, where the component fails to respond appropriately on specific input signal. As the time of the input request is assumed to be known, the unpredictability of the stochastic output response in this case is confined only to the signal value of the output, but not to the time of the response. This can be modeled in different ways but we will simply assume that the process is discrete by replacing the variable t in R(t,) with discrete time variable ti. Each component process as well as the system process will now become Bernoulli (discrete) processes, instead of being continuous, i.e. exponential for reliability or Poisson (i.e. renewal) process for availability calculation case. This also means that there is a finite probability that two or more components will fail simultaneously, given that the system is t-coherent. t-coherent means that the demand i is common, or simultaneous, for all the redundant components.

We may say that, in general, the reliability of the component can be modeled by a multiplicative combination of a continuous and discrete distribution

(4.3)

In case of a single component we have

(4.4)

where q, a probability of the failure on demand, is a small value. Hence,

(4.5)

##### where

(4.6)

and therefore the reliability model remains unchanged. Hence, the reliability parameters of interest have the same analytical expressions as without demand:

- Point-wise availability with failure rate = C+D and repair rate 

(4.7)

- Mean unavailability

(4.8)

- Unavailability with inspection interval  and zero actual repair-time

(4.9)

- Failure frequency

(4.10)

Unlike a single unit or a series system case, parallel, or redundant systems behave differently when the failure on demand is added to the continuous failures. Namely, the failure frequency of a redundant system has a threshold and does not decrease much if the inspection interval, or the test time, is smaller than the average time between demands, i.e.  < ti. Unavailability, however, continues to decrease infinitely. Recall that in a case of continuous failures, as we indicated by Equation [4.2], both unavailability and failure frequency can be constrained below any arbitrarily chosen values by means of suitably selected test time intervals . We have now changed this picture into the more realistic one: the failure frequency, in fact, cannot be brought to zero. Using a continuous model rationale, the failure on demand, therefore, acts as if it was a continuous failure with an additional continuous CCF part that was put in a series connection with an ordinary, i.e. non-demand continuous failure component. A look into a discrete model shows that what seemed to be a CCF originated from completely independent events. Instead of being dependent, we say that these events are coherent. In the next section we will introduce a more genuine common mode failures, the ones that are causally related, and therefore dependent.

## External Common Cause Failures

External common cause failures are those failures that are caused by the equipment’s surroundings. Strictly speaking, failures due to variation in environmental conditions do not belong to this group, and they may be referred to as “internal common cause failures”. The reason for this is that these conditions must exist, so that they are already incorporated in widely accepted standard reliability models such as MIL-217 or Bellcore. These conditions include, for example, ambient temperature, vibration, and moisture. External common cause failures are initiated by the events that in some cases may not necessarily exist, or in other cases may be completely avoided. External CCFs are more “random” in nature than internal CCFs.

The most important external CCFs in their order of importance include:

- Human errors

- Fire

- Earthquake

- Sabotage

- Terrorist attacks

- Airplane crashes

Human errors are undoubtedly the most important external CCFs. As statistics shows, human errors alone are likely to cause more detriment than all other failure events combined.

## Beta Factor Method

The beta factor method assumes that if the common cause failure were to occur all components in the CCF group would fail together. This simple method requires one parameter to be specified for each CCF group chosen. During the analysis each event in the CCF group is replaced by OR gate, resulting in adding the probability of the CCF event to the failure probability of the original event. Three CCF groups were used in the Safety Amplifier fault tree calculations. They are:

- +300V line group, CCF1 = MRY = {RY1, RY2, RY3, RY4}

- -300V line group, CCF2 = SRY = {RY5, RY6, RY7, RY8}

- fast scram group CCF3 = FSP = {V3, V4, V5, V6}

The reliability parameters of each group are chosen arbitrarily, and therefore are to be taken merely as a subjective estimate of some real values that are likely to occur.

If QA is the individual unavailability of the component A without CCF, and QCCF is the unavailability due to CCF, then the total unavailability QT is given by

(4.11)

where  is the beta factor for the given CCF group. Other CCF models provided by FaultTree+ include: Multiple Greek Letter (MGL), Alpha Factor Model, and Beta Binomial Failure Rate (BFR) Model (see Isograph Ltd., [1995]). All these models require additional parameters to be entered, which may introduce unacceptably high uncertainties and thus contradict the very method that was supposed to increase the accuracy. A further study may reduce these uncertainties. For the time being the Beta Factor Method is selected to give us the indication of an impact of CCF on the overall failure probability and other reliability parameters relative to the single mode failures, providing at the same time the minimum uncertainties. Our goal is not to calculate CCF with high accuracy, but to show that all single mode failures in the Safety Amplifier have, in fact, negligible impact on reliability compared to any plausible choice of CCF.