Chapter 5

# Results of the Fault Tree Analysis

## Component Reliability Data

### 5.1.1. Conversion Factors

Failure rates and repair rates, used for calculation in this report are expressed in units “per physical year” and inspection intervals in units “physical year”. When the particular number was originally given in units “per hour” the conversion factor of 4160 was used. When the number was given in units “per operating year” the conversion factor of 0.476 was used (see the example to follow). In case when the number was given in units “per working year” the conversion factor of 2.08 was used. Unit “per working year” is commonly used in radiology. By definition, one “working year” equals (50 weeks)x(5 days per week)x(8 hours per day) = 2000 hours. Terms “operating”, “working”, etc., can be moved from the left side to the right side of an equation. For example, the definition of one “working year” implies that one year equals (50 working weeks)x(5 working days per working week)x(8 working hours per working day) = 2000 working hours. The ratio of “one working hour per hour” is 0.2283.

The following procedures demonstrate how to perform the conversion factor calculations:

# Number of operating hours per working day = 16

# Number of working days per year = 52 weeks x 5 working days per week = 260

# Number of operating hours per year = 250 x 16 = 4160

Hence, the “number of operating hours per year” can also be expressed as the “number of hours per operating year”. Hence,

Similarly,

and also

### 5.1.2.  Reliability Database

The main source used for the component reliability data was [IAEA-TECDOC-478, 1988]. The record form of the data that includes failure rates, failure modes and component description, was combined with the failure model, name, and description of the component used in the FaultTree+ program to give the unique database used for reliability calculations.

Example:

Name used in project: UICCABLE

Description: Generic model type used for UIC1-C and UIC2-C (cable failures)

Source database: IAEA-TECDOC-478, “Component Reliability Data for Use in Probabilistic

Safety Assessment”

Name used in source database: CCSHE

Description: 	cable signal (supervisory) general

Component boundary: detail n/a

Operating mode: all

Operating environment: normal

Generic failure mode: short circuit

Original failure mode: short line to line

Failure rate: rec: 9.5E-7/hr, high: 1.7E-6/hr, low: 1.0E-8/hr

Source: IEEE 500 (1984) pg. 778

Ultimate source: expert opinion aggregation

Comment: Failure rate per 1000 circuit feet

Reference: IEEE 500 (1977)

All data provided is based on single expert opinion, aggregation of several non-nuclear sources, or nuclear experience. Sources used in the database include IEEE Standard 500, WASH-1400 Reactor Safety Study, and Swedish Reliability Data Book, and others. When the failure rate is provided in numbers or failures per demand, the corresponding standby failure rate per physical year and inspection time interval are used in the “Fixed unavailability and failure frequency” program entry model (see Isograph Ltd., 1995).

### Basic Event Model Construction

A basic event is an event that cannot be decomposed into other events. An example of a basic event is “cable will not fail up to time t given that initially it is in a success state”. It is modeled by exponential distribution with an estimated constant failure rate

(5.1)

The failure rate itself is estimated with some uncertainty. In this case we will associate either a normal or a log-normal distribution to the estimated parameter  of which the true value true we consider not to be fully known. The uncertainty of the estimate  is usually provided in the raw data as the interval (low,hi) for which P(true<low)<5% and P(true>hi)<5%. This can give us enough information to assess a standard deviation, or uncertainty. In the above case, however, the uncertainty was not provided in terms of confidence bounds but in terms of “low” and “high” probability rates that represent best and worst data points of the acquired sample. Since the number of data points is not known, the two values, low=1.0x10-8hr-1 and hi=1.7x10-6hr-1 cannot be used unambiguously. In cases like this, we will still assume that the two values represent the lower and upper 5% confidence bounds.

After the analysis is performed, the results with 90%, 95%, and 99% bounds of confidence are provided for the root event. The results for some cut sets and gates with high importances are also provided. Given that additional uncertainties, such as the one indicated above, are present, the resulting bounds of confidence should be taken with some reserve. This is, however, alleviated by the fact that the performed analysis was chosen to be conservative, whenever possible.

5.3.  List of Results

Table 5-1, Table 5-2, and Table 5-3 represent the main results of the fault tree analysis. Following these tables, the Safety Amplifier fault tree is presented in detail. Individual subsystem tree branches are presented on separate pages. Finally, the complete list of results of the analysis is provided.

Table 5-1 represents the root event summary. The root event can be described as an unsuccessful detachment of at least three shim rods. The scram signal may have propagated through fast scram line, slow scram line, or both. There are two different inputs. The first input is the power level scram, initiated by 125% full power. This input generates one event tree, and is associated to the output gate called SA-UIC (SA-UIC stands for “output Safety Amplifier – input Uncompensating Ionizing Chambers”). The initiating event for scram in this scenario comes from both uncompensating ionizing chambers located in the reactor core. The second input comes from the Log-N Amplifier. This is the period scram. The corresponding event tree is associated to the output gate called SA-LOGN (SA-LOGN stands for “output Safety Amplifier – input Log-N Amplifier”). The initiating event in this scenario is the short period signal from the Log-N Amplifier. This signal may trigger fast scram only.  It can be seen that the SA-UIC unavailability is lower than the SA-LOGN unavailability, but its failure frequency is higher.

| #### Gate Summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lower | Upper | Mean | Upper 90% | Upper 95% | Upper 99% |
| Gate : SA-UIC |  |  |  |  |  |  |  |
| Unavailability (Q) | 1.39E-06 | 1.25E-06 | 1.53E-06 | 1.35E-06 | 2.41E-06 | 2.71E-06 | 3.28E-06 |
| Frequency (w) | 6.60E-03 | 5.94E-03 | 7.26E-03 | 6.42E-03 | 1.15E-02 | 1.29E-02 | 1.56E-02 |
| Gate : SA-LOGN |  |  |  |  |  |  |  |
| Unavailability (Q) | 8.06E-06 | 7.25E-06 | 8.86E-06 | 7.92E-06 | 1.44E-05 | 1.62E-05 | 1.96E-05 |
| Frequency (w) | 4.17E-03 | 3.75E-03 | 4.59E-03 | 3.94E-03 | 5.42E-03 | 5.84E-03 | 6.62E-03 |

Table 5-1. Safety Amplifier Root Event Summary

As the Table 5-2 shows, the resulting reliability parameters are mainly determined by the failure events that are initiated by common causes. There are three common cause failure (CCF) groups included. These groups and associated reliability parameters are provided arbitrarily, and could be related to the intrinsic failures that are calculated more precisely, but have much less importance for the root failure. CCF1 group, that is a cut set for UIC input, for example, increases the UIC input unavailability by five orders of magnitude. CCF1 group includes relays at +300V line: RY1, RY2, RY3, and RY4 (see Figure 1-2). According to the results of the analysis, CCF1 and R49 failure are the two most important failures of the Safety Amplifier. Table 5-2 represents the list of the component failures with highest values of importance in the Safety Amplifier.

| Gate Importance Ranking |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Gate : SA-UIC SAFETY AMP FAILS TO UIC INPUT |  |  |  |  |  |  |
| Unavailability : 1.386e-6 Failure Frequency : 6.600e-3 |  |  |  |  |  |  |
| Event | Q | W | Fussell-Vesely | Birnbaum | B-Proschan | Sequential |
| CCF1 | 1.39E-06 | 6.60E-03 | 1 | 1 | 1 | 0 |
| CCF2 | 1.39E-06 | 6.60E-03 | 8.06E-06 | 8.06E-06 | 8.06E-06 | 8.76E-07 |
| R49 | 7.40E-06 | 8.70E-04 | 7.40E-06 | 1.39E-06 | 1.83E-07 | 7.40E-06 |
| CCF3 | 6.60E-07 | 3.30E-03 | 6.60E-07 | 1.39E-06 | 6.93E-07 | 6.60E-07 |
| Gate : SA-LOGN SAFETY AMP FAILS TO LOG-N INPUT |  |  |  |  |  |  |
| Unavailability : 8.055e-6 Failure Frequency : 4.170e-3 |  |  |  |  |  |  |
| Event | Q | W | Fussell-Vesely | Birnbaum | B-Proschan | Sequential |
| R49 | 7.40E-06 | 8.70E-04 | 9.18E-01 | 1 | 2.09E-01 | 0 |
| CCF3 | 6.60E-07 | 3.30E-03 | 8.19E-02 | 1 | 7.91E-01 | 0 |
| FSP V5 | 2.64E-06 | 3.30E-03 | 2.60E-06 | 7.92E-06 | 6.27E-06 | 6.27E-06 |
| FSP V6 | 2.64E-06 | 3.30E-03 | 8.65E-07 | 2.64E-06 | 2.09E-06 | 2.09E-06 |

Table 5-2. Safety Amplifier Root Importance Ranking

Table 5-3 contains the unavailabilities and failure frequencies of individual cut sets starting with the cut sets with the highest unavailability. Again, CCF1 and R49 are

| Gate Minimal Cut Set List |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Gate : SA-UIC SAFETY AMP FAILS TO UIC INPUT |  |  |  |  |  |  |
| Unavailability : 1.386e-6 Failure Frequency : 6.600e-3 |  |  |  |  |  |  |
| No | Q | W | Events |  |  |  |
| 1 | 1.39E-06 | 6.60E-03 | CCF1 |  |  |  |
| 2 | 1.03E-11 | 5.00E-08 | R49 | CCF2 |  |  |
| 3 | 9.15E-13 | 8.93E-09 | CCF3 | CCF2 |  |  |
| 4 | 5.06E-16 | 4.05E-12 | FIL 1A | FIL 1B | FIL 2A | FIL 2B |
| Gate : SA-LOGN SAFETY AMP FAILS TO LOG-N INPUT |  |  |  |  |  |  |
| Unavailability : 8.055e-6 Failure Frequency : 4.170e-3 |  |  |  |  |  |  |
| No | Q | W | Events |  |  |  |
| 1 | 7.40E-06 | 8.70E-04 | R49 |  |  |  |
| 2 | 6.60E-07 | 3.30E-03 | CCF3 |  |  |  |
| 3 | 6.97E-12 | 1.74E-08 | FSP V4 | FSP V5 |  |  |

Table 5-3. Minimal Cut Sets with Highest Unavailabilities

ranked at the top. R49 is a cut set for SA-LOGN but not for SA-UIC. In SA-UIC gate R49 is coupled with CCF2 to form a cut set. The complete list of results follows the subsequent figures that represent portions of the Safety Amplifier fault tree.

Figure 5-1. Top Portion of the Safety Amplifier Fault Tree (UIC Input)

Figure 5-2. Top Portion of the Safety Amplifier Fault Tree (Log-N Input)

Figure 5-3. FSP Subsystem Fault Tree Branch: 2.5 Out of 4 Logic

Figure 5-4. FSP Subsystem Fault Tree Branch (Simplified)

Figure 5-5. FST Subsystem Fault Tree Branch

Figure 5-6. SST Subsystem Fault Tree Branch (Collapsed)

Figure 5-7. SST Subsystem Fault Tree Branch (Expanded)

Figure 5-8. UIC Subsystem Fault Tree Branch

Figure 5-9. Safety Amplifier Fault Tree (UIC Input, Expanded)