STATISTICAL ESTIMATION OF THE UNAVAILABILITY OF THE

MNR SAFETY AMPLIFIER

By

IMRE VENCEL

A Thesis

Submitted to the School of Graduate Studies

in Partial Fulfillment of the Requirements

for the Degree

Master of Engineering

McMaster University

Outline

- The role of the SA

- Our objectives

- Keywords and definitions

- Methodology, mathematical assumptions,

and the techniques used

- Signal propagation chart

- The Fault Tree

- Uncertainties

- Results

- Conclusion

- Recommendations

#### The role of the SA

- MNR Safety Amplifier (SA) is the key component of the MNR safety system

- SA monitors the state of the core, processes the data, and performs the automatic core shutdown in case of emergency. It does it by following:

- SA keeps the safety rods attached to electromagnets during normal operation

- SA releases the rods when the scram conditions are met

#### Our Goal

- Calculate the main reliability parameters of the MNR Safety Amplifier (unavailability, failure frequency, importances of cut-sets)

#### Practical connotations

- the MNR safety report must be periodically updated for the purposes of the operating license renewal

Keywords and Definitions - 1

- Cut-set

Any event which, if happens, would guarantee the transition of the system to a failed state (a sufficient cause for failure).

- Minimal cut-set

A cut-set that has no nontrivial subsets that are cut-sets. The so-called “minimal cut-set method” allows us to calculate the reliability of the system by knowing only the reliabilities of the minimal cut-sets.

- Unavailability (Q):

##### The ratio between the down-time and the total time (= up-time + down-time)

Keywords and Definitions - 2

##### Failure frequency (t):

Probability that the unit would fail per unit time at time t. For all practical purposes t (hazard rate).

##### Stochastic Importance (IMP):

A measure of the sensitivity of the unavailability of the system with respect to the variation of the failure probability of a component

##### Common Cause Failure (CCF):

An event denoting simultaneous failure of

several units. For CCF, Pr(Xi=t1|Xj=t1)  0

- while Pr(Xi=t1) = 0.

Techniques used

- RAMS

- Fault Tree Analysis

Software used

- ™FaultTree+ 6.0 (Isograph Ltd.)

#### Notable difficulties

- Traditional (BP) reliability theory violates:

non-coherency, initial state assumptions, s-dependence, multi-input feature

- lack of data for CCF

Users of RAMS (FaultTree+)

# Hewlett Packard			Honewell

IBM						AT&T

Apple Computers			Lockheed Martin

McDonnell Douglas		Motorola

NASA					Nortel

Philips					Texas Instruments

Volkswagen				Westinghouse

US Nuclear Regulatory Commission

Mathematical assumptions

Failure-repair mechanism is modeled as a renewal process characterized by:

- Memory loss

## Markov processes; Poisson

## distributions of r.v.; underlying

## exponential life-time densities

## (at the component level)

- Rare event approximation

### Small hazard rates

Methodology

- Reverse engineering

- Modular Decomposition

- Build the fault tree from “top to bottom”

- Two views of the SA

- Operational view (schematic provided by the manufacturer)

- Safety view (requires different schematic)

Basic operation of the SA

- The function of the Safety Amp is to cutoff the current to at least three rod-magnets when: a) both UIC Chambers activate, or b) Log-N Amplifier activates.

# Figure 1. Fault trees of the Safety Amplifier

#### Uncertainty and related issues – part 1

- Minimal cut-set approximation (MCSA) is conservative in mean

- Bias of MCSA is shown to be small (MCSA adds a negligible small positive margin to the final result)

- FaultTree+ uses Monte Carlo method to calculate the confidence

- Log-Normal distributions may yield more adequate results than Normal distributions

#### Uncertainty and related issues – part 2

- As the level of redundancy increases, the sensitivity of the system undergoing repair increases (this is a price we pay for decreasing the unavailability)

- Importance measures were calculated in order to detect bottlenecks (Fussell-Vesely, Birnbaum, Barlow-Proschan importance)

- CCF’s have the highest uncertainties (variances), but, unfortunately, also have the highest importance measures

Common Cause Failures (CCF) can be dissected:

- By nature of origin (manmade, or decision-making related failures, and hardware, or random failures)

- By place of origin (external failures, internal failures)

- By nature of statistical correlation (coherent, cross-correlated)

Some standard ways to perform the CCF Analysis

- Beta Factor Model

- Multiple Greek Letter (MGL) Model

- Alpha Factor Model

- Beta binomial Failure Rate (BFR) Model

- Utilizing event trees

Selected CCF-related issues

- Elimination of the power supply (all active elements, e.g. relays, are normally energized)

- Operator error may cause multiple failures

- Failure on demand as a CCF (coherency, repairs, induced vs. spontaneous failures)

- External CCFs

- Human errors

- Fire

- Earthquake

- Sabotage

- Terrorist attacks

- Airplane crashes

#### Results: 1

| Gate Importance Ranking |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gate : SA-UIC SAFETY AMP FAILS TO UIC INPUT |  |  |  |  |  |  |  |  |  |  |
| Unavailability : 1.386e-6 Failure Frequency : 6.600e-3 |  |  |  |  |  |  |  |  |  |  |
| Event | Fussell-Vesely | Birnbaum |  | B-Proschan | Sequential |  |
| CCF1 | 1 | 1 |  | 1 | 0 |  |
| CCF2 | 8.06E-06 | 8.06E-06 |  | 8.06E-06 | 8.76E-07 |  |
| R49 | 7.40E-06 | 1.39E-06 |  | 1.83E-07 | 7.40E-06 |  |
| CCF3 | 6.60E-07 | 1.39E-06 |  | 6.93E-07 | 6.60E-07 |  |
| Gate : SA-LOGN SAFETY AMP FAILS TO LOG-N INPUT |  |  |  |  |  |  |  |  |  |  |  |
| Unavailability : 8.055e-6 Failure Frequency : 4.170e-3 |  |  |  |  |  |  |  |  |  |  |
| Event | Fussell-Vesely | Birnbaum |  | B-Proschan | Sequential |  |
| R49 | 9.18E-01 | 1 |  | 2.09E-01 | 0 |  |
| CCF3 | 8.19E-02 | 1 |  | 7.91E-01 | 0 |  |
| FSP V5 | 2.60E-06 | 7.92E-06 |  | 6.27E-06 | 6.27E-06 |  |
| FSP V6 | 8.65E-07 | 2.64E-06 |  | 2.09E-06 | 2.09E-06 |  |

Safety Amplifier Root Importance Ranking

#### Results: 2

| Gate Minimal Cut Set List |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gate : SA-UIC SAFETY AMP FAILS TO UIC INPUT |  |  |  |  |  |  |  |  |  |  |
| Unavailability : 1.386e-6 Failure Frequency : 6.600e-3 |  |  |  |  |  |  |  |  |  |  |
| No | Q | W | Events |  |  |  |
| 1 | 1.39E-06 | 6.60E-03 | CCF1 |  |  |  |
| 2 | 1.03E-11 | 5.00E-08 | R49 | CCF2 |  |  |
| 3 | 9.15E-13 | 8.93E-09 | CCF3 | CCF2 |  |  |
| 4 | 5.06E-16 | 4.05E-12 | FIL 1A | FIL 1B | FIL 2A | FIL 2B |
| Gate : SA-LOGN SAFETY AMP FAILS TO LOG-N INPUT |  |  |  |  |  |  |  |  |  |  |  |
| Unavailability : 8.055e-6 Failure Frequency : 4.170e-3 |  |  |  |  |  |  |  |  |  |  |
| No | Q | W | Events |  |  |  |
| 1 | 7.40E-06 | 8.70E-04 | R49 |  |  |  |
| 2 | 6.60E-07 | 3.30E-03 | CCF3 |  |  |  |
| 3 | 6.97E-12 | 1.74E-08 | FSP V4 | FSP V5 |  |  |

Minimal Cut Sets with Highest Unavailabilities

Conclusion

- With respect to a single failure mode the SA is inherently safe

- CCF probabilities are hard to calculate (censoring insufficiency, poor analytical models)

- R49 forms a bottleneck for the fast scram

Recommendations

- to gather more relevant statistical data, including internal data from the forty years of the MNR operating experience

- to apply Bayesian analysis for rare events

- to develop suitable models that can be applied to multi-input systems

- to develop suitable models for inspection optimization

- to develop suitable models that can handle different levels of supervision

- to develop suitable models that can account for the human errors

- to collect the relevant data and perform more accurate CCF analysis