| Student: | Test |
| --- | --- |
| Date: 2 May 2004 |  |
| Questions: |  |
| - Design Requirements (6 marks) - Explain the requirements for materials for reactor coolant and moderator, list the usual materials and discuss how they meet the requirements. (50%) - Explain how the type of the reactor influences the selection of the moderator and coolant material. (50%) - Power Reactor Types (6 marks) For the reactor shown in the figure below explain: - Draw a primary coolant flow diagram and explain the flow path. (30%) - Explain the configuration of the core and fuel assemblies. (30%) - List the most important difference compared to a CANDU reactor. (40%) - Process Design Evolution (5 marks) - Explain the main direction in CANDU process design evolution. (50%) - Capture the most important changes to the ACR-700 design as compared to the traditional CANDU. (50%) |  |

- Flow Instabilities (5 marks) For the flow instability diagram shown in the figure below: - Explain the instability criterion. (30%) - Discuss the effect of channel heat transfer to the instability curve. (30%) - Discuss the importance in case of flow diversion in a loop with two parallel non-equal channels. (40%) - Thermal Efficiency (6 marks) - Define the thermal efficiency of a NPP and explain the factors of influence. (50%) - Explain the methods (design options) to increase NPP thermal efficiency, and discuss feasibility and importance of each option. (50%) - Steam Generator Calculations (6 marks) - Explain the principles of the design of the steam generator (40%) - Explain the behavior of the steam generator using the heat duty diagram (60%) - Fuel-Coolant Heat Transfer (6 marks) - Draw a diagram of temperature distribution across the fuel element and coolant in radial direction, and explain the change of rate of heat transfer across different components (50%) - Draw a diagram of temperature distribution along the fuel element and coolant in axial direction and explain the change of rate of heat transfer along different components (50%)

- Two-phase Flow (5 marks) - Identify all boiling heat-transfer modes and transition points in the boiling curve. (40%) - State all possible boiling heat-transfer modes encountered in a heat-flux-controlled system with increasing heat flux from single-phase liquid flow at the same local flow conditions (i.e., pressure, mass flux and quality). (25%) - State all possible boiling heat-transfer modes encountered in a temperature-controlled system with increasing wall temperature from single-phase liquid flow at the same local flow conditions (i.e., pressure, mass flux and quality). (35%) - Film Boiling (5 marks) - Identify heat-transfer modes (i.e., conduction, convection, and radiation) between the heated wall and two phases in dispersed-flow film boiling. (40%) - Provide the direction of heat flow at each solid line. (20%) - What does the dashed line represent? (40%)

- Pressure Drop (7 marks) - Identify all pressure-drop components with flow over a string of heated 37-element bundles inside a horizontal pressure tube. (20%) - Arrange thermodynamic qualities for the following transition points in ascending order: (20%) - Onset of annular flow - Onset of nucleate boiling - Onset of significant void - Saturation liquid point - Saturation vapour point - Calculate the two-phase pressure drop for light-water flow over the 37-element bundle (element outside diameter of 13.06 mm) inside a pressure tube (inside diameter of 103.86 mm) (60%) Pressure: 9 MPa Mass flow rate: 17 kg/s Mass quality: 0.2 Flow area: 3515 mm2 Bundle loss coefficient: 1.7 Saturated liquid density: 704.7 kg/m3 Saturated vapour density: 48.8 kg/m3 Two-phase multiplier, 2LO = (1 + x (f / g - 1))

- CHF (7 marks) - Evaluate the critical heat flux, using the attached CHF table, for the 37-element bundle (element outside diameter of 13.06 mm) inside a pressure tube (inside diameter of 103.86 mm) at the pressure of 9.5 MPa, mass flow rate of 17 kg/s and thermodynamic quality of 20%. (50%) - Evaluate the critical heat flux for the bundle inside a pressure tube with 5% diametral creep (i.e., pressure-tube inside diameter is 5% larger) at the same flow conditions. (25%) - Identify the boiling heat-transfer modes for these two cases if the local heat flux is 1500 kW/m2. (25%) Pressure (kPa); Mass Flux (kg.m-2.s-1); Quality. _0.5; _0.4; _0.3; _0.2; _0.15; _0.1; _0.05; 0; 0.05; 0.1; 0.15; 0.2; 0.25; 0.3; 0.35; 0.4; 0.45; 0.5; 0.6; 0.7; 0.8; 0.9; 1. 9000; 0; 4836; 4571; 4301; 4034; 3902; 3783; 3639; 3352; 2503; 1977; 1637; 1290; 1179; 992; 843; 739; 660; 608; 560; 471; 431; 349; 0. 9000; 50; 5422; 5114; 4813; 4547; 4432; 4335; 4213; 3960; 3071; 2528; 2304; 2110; 1945; 1810; 1699; 1610; 1536; 1457; 1152; 971; 938; 650; 0. 9000; 100; 5916; 5571; 5251; 4988; 4892; 4811; 4704; 4462; 3658; 3134; 2976; 2841; 2697; 2575; 2458; 2373; 2317; 2165; 1554; 1306; 1300; 1040; 0. 9000; 300; 6366; 5916; 5527; 5300; 5260; 5239; 5236; 5194; 4944; 4270; 3692; 3478; 3344; 3257; 3065; 2894; 2713; 2334; 1672; 1380; 1197; 899; 0. 9000; 500; 6641; 6100; 5668; 5412; 5332; 5205; 5147; 5114; 4925; 4243; 3625; 3364; 3160; 2999; 2887; 2741; 2351; 1983; 1583; 1282; 904; 626; 0. 9000; 1000; 7221; 6432; 5947; 5625; 5398; 4936; 4705; 4703; 4697; 4204; 3520; 3118; 2854; 2661; 2545; 2118; 1553; 1143; 984; 729; 346; 311; 0. 9000; 1500; 7976; 7023; 6356; 5856; 5535; 4958; 4693; 4467; 4189; 3730; 3227; 2850; 2712; 2495; 2034; 1429; 1081; 892; 546; 350; 131; 119; 0. 9000; 2000; 9111; 8171; 7160; 6258; 5863; 5299; 4835; 4196; 3569; 3097; 2746; 2448; 2312; 1931; 1428; 1027; 745; 603; 345; 186; 97; 48; 0. 9000; 2500; 10108; 9252; 8185; 7162; 6749; 6025; 5004; 4119; 3339; 2797; 2521; 2241; 1942; 1373; 1028; 809; 570; 348; 211; 136; 96; 48; 0. 9000; 3000; 10887; 10041; 8998; 7755; 7210; 6399; 5097; 4045; 3240; 2604; 2289; 2038; 1724; 1354; 967; 708; 508; 393; 282; 169; 103; 49; 0. 9000; 3500; 11622; 10683; 9660; 8227; 7395; 6453; 5140; 3995; 3155; 2487; 2124; 1896; 1643; 1276; 952; 650; 457; 419; 305; 184; 107; 54; 0. 9000; 4000; 12336; 11283; 10200; 8756; 7669; 6674; 5266; 3983; 3066; 2349; 1931; 1750; 1604; 1375; 982; 664; 558; 499; 331; 190; 108; 55; 0. 9000; 4500; 13022; 11857; 10680; 9235; 7924; 6936; 5469; 4047; 3026; 2245; 1827; 1719; 1631; 1385; 1037; 771; 670; 615; 381; 205; 109; 56; 0. 9000; 5000; 13683; 12418; 11127; 9685; 8259; 7305; 5645; 4183; 3080; 2249; 1836; 1755; 1663; 1416; 1152; 908; 816; 717; 445; 239; 116; 57; 0. 9000; 5500; 14332; 12927; 11580; 10180; 8872; 7818; 5764; 4272; 3210; 2355; 1963; 1856; 1760; 1542; 1290; 1071; 935; 801; 512; 281; 130; 62; 0. 9000; 6000; 14961; 13402; 11998; 10758; 9759; 8777; 6149; 4331; 3303; 2421; 2126; 2045; 1937; 1699; 1500; 1219; 1072; 892; 578; 329; 157; 73; 0. 9000; 6500; 15570; 13909; 12387; 11336; 10230; 9257; 6353; 4416; 3344; 2492; 2346; 2317; 2197; 1985; 1710; 1431; 1170; 977; 646; 378; 186; 88; 0. 9000; 7000; 16165; 14403; 12779; 11578; 10702; 9737; 6355; 4542; 3482; 2788; 2527; 2402; 2245; 2070; 1785; 1497; 1258; 1058; 712; 426; 216; 103; 0. 9000; 7500; 16748; 14881; 13166; 11824; 10747; 9739; 6394; 4637; 3660; 3076; 2877; 2745; 2567; 2142; 1800; 1557; 1339; 1140; 778; 475; 248; 118; 0. 9000; 8000; 17323; 15340; 13551; 12107; 10954; 9741; 6556; 4714; 3838; 3419; 3342; 3299; 3162; 2736; 2042; 1620; 1420; 1218; 842; 525; 281; 134; 0. 10000; 0; 4568; 4337; 4103; 3874; 3760; 3658; 3534; 3274; 2464; 1922; 1596; 1276; 1163; 991; 849; 748; 680; 625; 562; 474; 410; 282; 0. 10000; 50; 5126; 4857; 4591; 4336; 4230; 4146; 4040; 3814; 3002; 2482; 2256; 2057; 1890; 1750; 1641; 1555; 1482; 1415; 1139; 965; 886; 645; 0. 10000; 100; 5597; 5296; 5007; 4734; 4637; 4565; 4472; 4253; 3512; 3015; 2865; 2728; 2582; 2457; 2347; 2266; 2213; 2105; 1551; 1296; 1211; 897; 0. 10000; 300; 6039; 5642; 5266; 4961; 4891; 4870; 4868; 4832; 4636; 4016; 3506; 3393; 3315; 3233; 2987; 2781; 2593; 2240; 1383; 1076; 1021; 674; 0. 10000; 500; 6318; 5831; 5417; 5074; 4965; 4855; 4800; 4765; 4599; 4013; 3353; 3165; 3038; 2871; 2644; 2239; 2002; 1651; 1137; 848; 796; 522; 0. 10000; 1000; 6912; 6184; 5732; 5328; 5073; 4676; 4463; 4376; 4328; 3822; 3225; 2936; 2680; 2484; 2171; 1342; 1033; 802; 636; 460; 298; 250; 0. 10000; 1500; 7650; 6747; 6118; 5570; 5212; 4712; 4476; 4178; 3843; 3402; 2958; 2559; 2393; 2102; 1655; 938; 647; 547; 451; 253; 127; 86; 0. 10000; 2000; 8720; 7826; 6896; 6030; 5591; 5000; 4575; 3974; 3349; 2872; 2468; 2106; 1912; 1402; 972; 677; 480; 355; 303; 155; 95; 46; 0. 10000; 2500; 9664; 8829; 7895; 7010; 6589; 5726; 4678; 3901; 3191; 2623; 2258; 1901; 1601; 1035; 666; 533; 399; 280; 195; 126; 95; 47; 0. 10000; 3000; 10400; 9580; 8648; 7587; 7077; 6149; 4779; 3891; 3089; 2444; 2030; 1701; 1419; 1001; 647; 478; 388; 300; 256; 147; 97; 48; 0. 10000; 3500; 11099; 10200; 9259; 8008; 7253; 6254; 4797; 3830; 3005; 2327; 1877; 1591; 1361; 1062; 723; 525; 414; 364; 274; 151; 99; 51; 0. 10000; 4000; 11783; 10775; 9774; 8533; 7582; 6503; 4841; 3728; 2909; 2178; 1686; 1470; 1345; 1147; 870; 641; 492; 429; 291; 160; 101; 53; 0. 10000; 4500; 12444; 11327; 10230; 8989; 7824; 6787; 4940; 3754; 2892; 2087; 1605; 1476; 1410; 1262; 1016; 768; 590; 545; 359; 188; 105; 54; 0. 10000; 5000; 13081; 11867; 10655; 9402; 8114; 7092; 5116; 3898; 2977; 2088; 1612; 1562; 1539; 1383; 1130; 876; 686; 639; 430; 229; 112; 56; 0. 10000; 5500; 13694; 12355; 11078; 9834; 8616; 7476; 5265; 4034; 3114; 2208; 1742; 1692; 1663; 1496; 1261; 993; 796; 703; 490; 274; 126; 59; 0. 10000; 6000; 14268; 12772; 11467; 10320; 9342; 8224; 5687; 4147; 3227; 2286; 1927; 1902; 1821; 1641; 1440; 1183; 993; 842; 553; 317; 150; 70; 0. 10000; 6500; 14835; 13236; 11837; 10808; 9713; 8600; 5912; 4241; 3275; 2378; 2206; 2200; 2098; 1895; 1687; 1382; 1133; 942; 620; 363; 179; 85; 0. 10000; 7000; 15399; 13708; 12204; 11028; 10083; 8975; 5926; 4267; 3390; 2707; 2422; 2309; 2182; 2003; 1720; 1432; 1212; 1021; 686; 410; 208; 99; 0. 10000; 7500; 15952; 14166; 12567; 11253; 10103; 8977; 5940; 4279; 3559; 2995; 2729; 2594; 2483; 2092; 1733; 1484; 1287; 1097; 750; 459; 240; 114; 0. 10000; 8000; 16496; 14588; 12938; 11536; 10351; 8979; 5955; 4484; 3783; 3325; 3120; 2988; 2864; 2621; 2015; 1596; 1393; 1182; 812; 507; 272; 129; 0.

- Basic Conservation Equations for Two-Fluid Models (6 marks) Write the microscopic and macroscopic form of the conservation equations and explain the role of each term. - CATHENA Computer Code (6 marks) - Explain the most important features of the CATHENA thermal-hydraulic model (model characteristics, conservation equations, flow regime map, nodalization, which parameters are calculated, etc.) (50%) - Explain the most important features of the CATHENA heat transfer model (which models are available, which parameters are calculated, linkage to hydraulics, correlations etc.) (50%) - Thermal-hydraulic Network Calculations (6 marks) Explain the advantages and disadvantages of different numerical schemes: explicit, semi-implicit (implicit in flow and pressure, explicit in mass and enthalpy), and fully implicit. - Nodalization (6 marks) - Write the mass equations for the thermal-hydraulic system below and show it in matrix form. Bonus for high mark: Write the momentum equations for the thermal-hydraulic system above and show it in matrix form.