CHF example:

Calculate the CHF for vertical upward flow of water inside a uniformly heated tube (0.012 m ID) at the following conditions:

Pressure:	8 200 kPa

Mass Flux:	5 000 kg.m-2.s-1

Quality:	0.175

Solution:

(a)	All correction factors are equal to 1 except for K1

K1=0.008D12=0.0080.01212=0.82

(Note that the heated length is assumed to be sufficiently long such that K4 = 1).

(b)	Evaluate the CHF value for an 8 mm tube based on the same conditions, with the CHF table

P	 G		 X		CHF8 mm

(kPa)	(kg.m-2.s-1)			(kW.m-2)

From table		8 000	   5 000	0.15		2 120

8 000	   5 000	0.20		2 031

From interpolation	8 000	   5 000	0.175		2 075.5

From table		9 000	   5 000	0.15		1 836

9 000	   5 000	0.20		1 755

From interpolation	9 000	   5 000	0.175		1 795.5

From interpolation	8 200	   5 000	0.175		2 019.5

(c)	Correct for the diameter effect

CHF12 mm=CHF8 mm K1=2019.5 (0.82)=1656 kW.m−2

CHF (in kW.m-2) Look-Up Table for Light Water in Tubes (LW-T-96)

| Pressure (kPa) | Mass Flux (kg.m-2.s-1) | Quality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | _0.5 | _0.4 | _0.3 | _0.2 | _0.15 | _0.1 | _0.05 | 0 | 0.05 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0.35 | 0.4 | 0.45 | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | 1 |
| 6000 | 0 | 5626 | 5219 | 4798 | 4387 | 4185 | 3995 | 3777 | 3415 | 2669 | 2291 | 1859 | 1416 | 1150 | 941 | 787 | 686 | 608 | 547 | 439 | 348 | 344 | 327 | 0 |
| 6000 | 50 | 6300 | 5838 | 5443 | 5092 | 4920 | 4760 | 4575 | 4272 | 3564 | 3191 | 2812 | 2431 | 2204 | 2023 | 1871 | 1749 | 1645 | 1537 | 1229 | 989 | 942 | 823 | 0 |
| 6000 | 100 | 6873 | 6366 | 5999 | 5708 | 5567 | 5434 | 5273 | 4973 | 4079 | 3530 | 3377 | 3237 | 3089 | 2966 | 2854 | 2724 | 2598 | 2431 | 1833 | 1647 | 1511 | 1070 | 0 |
| 6000 | 300 | 7318 | 6710 | 6451 | 6395 | 6379 | 6330 | 6310 | 6255 | 5942 | 5126 | 4783 | 4679 | 4496 | 4269 | 4066 | 3800 | 3584 | 3283 | 2663 | 2330 | 1766 | 1193 | 0 |
| 6000 | 500 | 7573 | 6883 | 6586 | 6512 | 6480 | 6364 | 6316 | 6261 | 5978 | 5371 | 5005 | 4822 | 4683 | 4333 | 4077 | 3812 | 3707 | 3468 | 2874 | 2374 | 1636 | 1061 | 0 |
| 6000 | 1000 | 8080 | 7185 | 6742 | 6576 | 6502 | 6256 | 6114 | 6008 | 5633 | 5334 | 4857 | 4429 | 4177 | 3788 | 3528 | 3418 | 3263 | 2964 | 1965 | 1257 | 803 | 735 | 0 |
| 6000 | 1500 | 8817 | 7758 | 7023 | 6667 | 6585 | 6331 | 6146 | 5787 | 5138 | 4703 | 4326 | 3964 | 3637 | 3309 | 3056 | 2839 | 2550 | 2068 | 1029 | 785 | 714 | 707 | 0 |
| 6000 | 2000 | 10109 | 9053 | 7842 | 6970 | 6796 | 6559 | 6167 | 5531 | 4716 | 4227 | 3875 | 3532 | 3229 | 2919 | 2640 | 2383 | 1789 | 1096 | 532 | 518 | 439 | 435 | 0 |
| 6000 | 2500 | 11237 | 10324 | 8947 | 7698 | 7356 | 7038 | 6235 | 5383 | 4543 | 4030 | 3545 | 3244 | 3054 | 2797 | 2311 | 1632 | 930 | 519 | 370 | 305 | 107 | 52 | 0 |
| 6000 | 3000 | 12123 | 11219 | 9913 | 8360 | 7693 | 7264 | 6241 | 5237 | 4449 | 3882 | 3340 | 3043 | 2876 | 2560 | 1887 | 1093 | 571 | 477 | 472 | 353 | 122 | 52 | 0 |
| 6000 | 3500 | 12969 | 11949 | 10669 | 8979 | 7986 | 7458 | 6243 | 4962 | 4218 | 3651 | 3219 | 2961 | 2696 | 2235 | 1522 | 871 | 582 | 576 | 505 | 382 | 145 | 57 | 0 |
| 6000 | 4000 | 13791 | 12626 | 11288 | 9594 | 8324 | 7679 | 6245 | 4590 | 3787 | 3318 | 3038 | 2893 | 2570 | 1995 | 1376 | 988 | 952 | 952 | 580 | 384 | 146 | 59 | 0 |
| 6000 | 4500 | 14582 | 13274 | 11857 | 10119 | 8549 | 7682 | 6247 | 4578 | 3624 | 3133 | 2922 | 2814 | 2470 | 1863 | 1358 | 1154 | 1133 | 1124 | 622 | 386 | 147 | 60 | 0 |
| 6000 | 5000 | 15341 | 13920 | 12372 | 10555 | 8820 | 7811 | 6336 | 4701 | 3674 | 3115 | 2918 | 2795 | 2432 | 1792 | 1319 | 1237 | 1183 | 1130 | 650 | 390 | 148 | 64 | 0 |
| 6000 | 5500 | 16091 | 14489 | 12910 | 11165 | 9671 | 8416 | 6533 | 4793 | 3751 | 3148 | 2936 | 2816 | 2475 | 1877 | 1459 | 1363 | 1345 | 1149 | 698 | 411 | 167 | 74 | 0 |
| 6000 | 6000 | 16823 | 15039 | 13396 | 12151 | 10623 | 9568 | 7356 | 5118 | 3824 | 3169 | 2946 | 2843 | 2506 | 2049 | 1755 | 1621 | 1405 | 1192 | 756 | 438 | 196 | 90 | 0 |
| 6000 | 6500 | 17524 | 15635 | 13826 | 12525 | 11575 | 10720 | 8043 | 5370 | 3900 | 3211 | 2979 | 2886 | 2586 | 2247 | 2026 | 1760 | 1468 | 1250 | 818 | 472 | 223 | 105 | 0 |
| 6000 | 7000 | 18210 | 16212 | 14268 | 12743 | 11838 | 11239 | 8441 | 5556 | 4010 | 3305 | 3095 | 2988 | 2795 | 2492 | 2169 | 1822 | 1545 | 1314 | 887 | 528 | 260 | 123 | 0 |
| 6000 | 7500 | 18884 | 16771 | 14706 | 13013 | 12063 | 11299 | 8839 | 5779 | 4287 | 3660 | 3411 | 3187 | 2980 | 2635 | 2268 | 1901 | 1632 | 1394 | 960 | 587 | 299 | 142 | 0 |
| 6000 | 8000 | 19548 | 17309 | 15140 | 13347 | 12447 | 11537 | 9144 | 6019 | 4481 | 3966 | 3675 | 3423 | 3262 | 2860 | 2359 | 1981 | 1717 | 1479 | 1034 | 646 | 341 | 162 | 0 |
| 7000 | 0 | 5361 | 5010 | 4651 | 4293 | 4118 | 3954 | 3762 | 3426 | 2692 | 2336 | 1918 | 1485 | 1212 | 996 | 834 | 723 | 641 | 575 | 466 | 369 | 357 | 339 | 0 |
| 7000 | 50 | 6002 | 5599 | 5236 | 4926 | 4778 | 4644 | 4483 | 4198 | 3498 | 3137 | 2769 | 2397 | 2165 | 1981 | 1826 | 1706 | 1604 | 1501 | 1200 | 984 | 940 | 774 | 0 |
| 7000 | 100 | 6539 | 6094 | 5738 | 5474 | 5355 | 5245 | 5105 | 4818 | 3957 | 3401 | 3239 | 3093 | 2941 | 2820 | 2707 | 2595 | 2489 | 2302 | 1710 | 1515 | 1479 | 1060 | 0 |
| 7000 | 300 | 6998 | 6441 | 6104 | 6015 | 6002 | 5947 | 5886 | 5865 | 5582 | 4834 | 4494 | 4247 | 4046 | 3862 | 3632 | 3430 | 3318 | 3065 | 2402 | 2144 | 1735 | 1179 | 0 |
| 7000 | 500 | 7264 | 6617 | 6233 | 6123 | 6088 | 5953 | 5940 | 5887 | 5690 | 5134 | 4682 | 4316 | 4157 | 3900 | 3634 | 3469 | 3366 | 3157 | 2596 | 2213 | 1587 | 1029 | 0 |
| 7000 | 1000 | 7798 | 6930 | 6386 | 6216 | 6135 | 5799 | 5604 | 5505 | 5318 | 5070 | 4472 | 3892 | 3626 | 3347 | 3136 | 3031 | 3028 | 2838 | 1774 | 1121 | 735 | 613 | 0 |
| 7000 | 1500 | 8557 | 7520 | 6715 | 6339 | 6253 | 5886 | 5603 | 5145 | 4673 | 4301 | 3874 | 3486 | 3189 | 2964 | 2735 | 2523 | 2250 | 1728 | 805 | 488 | 273 | 259 | 0 |
| 7000 | 2000 | 9793 | 8774 | 7597 | 6676 | 6480 | 6142 | 5684 | 4952 | 4275 | 3785 | 3407 | 3122 | 2890 | 2731 | 2451 | 2023 | 1445 | 844 | 432 | 322 | 196 | 190 | 0 |
| 7000 | 2500 | 10882 | 9986 | 8709 | 7496 | 7142 | 6690 | 5806 | 4876 | 4104 | 3537 | 3147 | 2922 | 2723 | 2445 | 1983 | 1367 | 789 | 424 | 261 | 204 | 99 | 51 | 0 |
| 7000 | 3000 | 11730 | 10850 | 9620 | 8170 | 7523 | 6953 | 5816 | 4724 | 3981 | 3369 | 2940 | 2714 | 2491 | 2133 | 1570 | 967 | 553 | 425 | 346 | 263 | 112 | 52 | 0 |
| 7000 | 3500 | 12535 | 11558 | 10344 | 8740 | 7757 | 7135 | 5848 | 4567 | 3834 | 3199 | 2786 | 2565 | 2294 | 1896 | 1323 | 782 | 519 | 490 | 409 | 317 | 135 | 57 | 0 |
| 7000 | 4000 | 13317 | 12216 | 10929 | 9320 | 8041 | 7398 | 5938 | 4372 | 3469 | 2928 | 2645 | 2490 | 2201 | 1730 | 1228 | 840 | 734 | 732 | 470 | 317 | 136 | 58 | 0 |
| 7000 | 4500 | 14070 | 12839 | 11469 | 9769 | 8188 | 7399 | 6061 | 4410 | 3347 | 2743 | 2515 | 2418 | 2139 | 1640 | 1200 | 930 | 872 | 854 | 492 | 317 | 137 | 59 | 0 |
| 7000 | 5000 | 14792 | 13465 | 11954 | 10124 | 8354 | 7427 | 6225 | 4552 | 3378 | 2696 | 2458 | 2380 | 2105 | 1591 | 1193 | 1010 | 949 | 888 | 521 | 326 | 138 | 63 | 0 |
| 7000 | 5500 | 15509 | 14000 | 12474 | 10713 | 9223 | 8025 | 6409 | 4682 | 3454 | 2710 | 2471 | 2382 | 2146 | 1673 | 1308 | 1154 | 1102 | 952 | 582 | 348 | 153 | 70 | 0 |
| 7000 | 6000 | 16208 | 14521 | 12931 | 11464 | 10460 | 9338 | 7226 | 4752 | 3483 | 2714 | 2470 | 2415 | 2213 | 1840 | 1539 | 1385 | 1228 | 1032 | 655 | 379 | 179 | 84 | 0 |
| 7000 | 6500 | 16875 | 15091 | 13336 | 12214 | 11432 | 10650 | 7804 | 4867 | 3535 | 2784 | 2519 | 2466 | 2330 | 2140 | 1879 | 1599 | 1311 | 1108 | 725 | 422 | 210 | 99 | 0 |
| 7000 | 7000 | 17529 | 15640 | 13763 | 12432 | 11718 | 11183 | 8065 | 5055 | 3745 | 3006 | 2708 | 2595 | 2464 | 2260 | 2014 | 1687 | 1401 | 1186 | 795 | 476 | 243 | 116 | 0 |
| 7000 | 7500 | 18170 | 16174 | 14182 | 12682 | 11740 | 11185 | 8202 | 5208 | 3974 | 3339 | 3032 | 2913 | 2779 | 2541 | 2199 | 1773 | 1489 | 1269 | 866 | 530 | 277 | 132 | 0 |
| 7000 | 8000 | 18806 | 16673 | 14610 | 12995 | 12067 | 11187 | 8424 | 5405 | 4172 | 3727 | 3483 | 3384 | 3259 | 2846 | 2288 | 1851 | 1576 | 1353 | 936 | 581 | 312 | 149 | 0 |
| 8000 | 0 | 5101 | 4795 | 4484 | 4175 | 4022 | 3883 | 3716 | 3403 | 2547 | 2044 | 1674 | 1295 | 1195 | 994 | 838 | 731 | 653 | 591 | 542 | 456 | 423 | 364 | 0 |
| 8000 | 50 | 5714 | 5359 | 5025 | 4744 | 4615 | 4503 | 4362 | 4081 | 3170 | 2622 | 2374 | 2169 | 2006 | 1871 | 1761 | 1667 | 1579 | 1490 | 1183 | 980 | 938 | 720 | 0 |
| 8000 | 100 | 6229 | 5834 | 5487 | 5235 | 5132 | 5039 | 4916 | 4650 | 3796 | 3239 | 3086 | 2969 | 2837 | 2708 | 2601 | 2507 | 2417 | 2281 | 1678 | 1457 | 1449 | 1050 | 0 |
| 8000 | 300 | 6685 | 6179 | 5792 | 5654 | 5637 | 5577 | 5512 | 5484 | 5250 | 4549 | 4207 | 3911 | 3759 | 3642 | 3458 | 3328 | 3212 | 2893 | 2144 | 1877 | 1538 | 1083 | 0 |
| 8000 | 500 | 6958 | 6354 | 5920 | 5763 | 5718 | 5605 | 5597 | 5544 | 5324 | 4784 | 4306 | 3829 | 3583 | 3411 | 3248 | 3096 | 2964 | 2629 | 2068 | 1858 | 1427 | 917 | 0 |
| 8000 | 1000 | 7518 | 6668 | 6144 | 5919 | 5765 | 5314 | 5072 | 5040 | 4997 | 4755 | 4106 | 3452 | 3103 | 2847 | 2749 | 2560 | 2331 | 2069 | 1413 | 944 | 554 | 511 | 0 |
| 8000 | 1500 | 8280 | 7266 | 6556 | 6124 | 5911 | 5334 | 5042 | 4774 | 4459 | 4066 | 3562 | 3137 | 2863 | 2639 | 2355 | 2001 | 1661 | 1209 | 643 | 364 | 205 | 195 | 0 |
| 8000 | 2000 | 9465 | 8470 | 7416 | 6490 | 6179 | 5626 | 5178 | 4623 | 4015 | 3483 | 3039 | 2749 | 2551 | 2345 | 2014 | 1559 | 1068 | 656 | 388 | 250 | 154 | 134 | 0 |
| 8000 | 2500 | 10508 | 9633 | 8457 | 7323 | 6937 | 6335 | 5386 | 4575 | 3816 | 3144 | 2769 | 2551 | 2319 | 1899 | 1514 | 1099 | 655 | 396 | 237 | 178 | 96 | 49 | 0 |
| 8000 | 3000 | 11320 | 10465 | 9316 | 7976 | 7376 | 6702 | 5426 | 4348 | 3609 | 2906 | 2535 | 2340 | 2093 | 1694 | 1253 | 811 | 524 | 397 | 307 | 225 | 111 | 50 | 0 |
| 8000 | 3500 | 12091 | 11136 | 10017 | 8489 | 7576 | 6794 | 5461 | 4205 | 3479 | 2798 | 2381 | 2179 | 1947 | 1616 | 1128 | 696 | 501 | 470 | 385 | 258 | 122 | 55 | 0 |
| 8000 | 4000 | 12839 | 11763 | 10584 | 9021 | 7836 | 7014 | 5626 | 4145 | 3241 | 2592 | 2238 | 2084 | 1893 | 1528 | 1063 | 702 | 603 | 577 | 401 | 258 | 122 | 56 | 0 |
| 8000 | 4500 | 13559 | 12361 | 11093 | 9494 | 8078 | 7215 | 5921 | 4248 | 3137 | 2403 | 2116 | 2037 | 1867 | 1475 | 1066 | 783 | 738 | 676 | 425 | 259 | 122 | 57 | 0 |
| 8000 | 5000 | 14251 | 12945 | 11568 | 9938 | 8351 | 7424 | 6106 | 4394 | 3169 | 2420 | 2120 | 2031 | 1821 | 1450 | 1147 | 923 | 859 | 756 | 470 | 275 | 125 | 59 | 0 |
| 8000 | 5500 | 14934 | 13466 | 12056 | 10488 | 9083 | 7990 | 6213 | 4479 | 3283 | 2478 | 2177 | 2070 | 1878 | 1570 | 1300 | 1076 | 988 | 850 | 535 | 304 | 139 | 65 | 0 |
| 8000 | 6000 | 15597 | 13983 | 12477 | 11140 | 10142 | 9265 | 6851 | 4526 | 3352 | 2510 | 2237 | 2159 | 1998 | 1721 | 1519 | 1256 | 1136 | 938 | 607 | 346 | 165 | 77 | 0 |
| 8000 | 6500 | 16235 | 14521 | 12879 | 11791 | 10678 | 9903 | 7172 | 4629 | 3419 | 2556 | 2432 | 2391 | 2207 | 2027 | 1794 | 1514 | 1225 | 1025 | 678 | 396 | 197 | 93 | 0 |
| 8000 | 7000 | 16859 | 15043 | 13288 | 12019 | 11214 | 10542 | 7253 | 4766 | 3593 | 2843 | 2558 | 2466 | 2287 | 2179 | 1832 | 1585 | 1316 | 1111 | 747 | 447 | 228 | 108 | 0 |
| 8000 | 7500 | 17470 | 15550 | 13689 | 12260 | 11235 | 10544 | 7292 | 4874 | 3748 | 3136 | 2935 | 2820 | 2611 | 2287 | 1980 | 1654 | 1402 | 1193 | 814 | 498 | 260 | 124 | 0 |
| 8000 | 8000 | 18075 | 16026 | 14098 | 12584 | 11555 | 10546 | 7482 | 4967 | 3931 | 3490 | 3380 | 3376 | 3257 | 2798 | 2157 | 1718 | 1479 | 1272 | 881 | 548 | 295 | 140 | 0 |

CHF (in kW.m-2) Look-Up Table for Light Water in Tubes (LW-T-96)

| Pressure (kPa) | Mass Flux (kg.m-2.s-1) | Quality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | _0.5 | _0.4 | _0.3 | _0.2 | _0.15 | _0.1 | _0.05 | 0 | 0.05 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0.35 | 0.4 | 0.45 | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | 1 |
| 9000 | 0 | 4836 | 4571 | 4301 | 4034 | 3902 | 3783 | 3639 | 3352 | 2503 | 1977 | 1637 | 1290 | 1179 | 992 | 843 | 739 | 660 | 608 | 560 | 471 | 431 | 349 | 0 |
| 9000 | 50 | 5422 | 5114 | 4813 | 4547 | 4432 | 4335 | 4213 | 3960 | 3071 | 2528 | 2304 | 2110 | 1945 | 1810 | 1699 | 1610 | 1536 | 1457 | 1152 | 971 | 938 | 650 | 0 |
| 9000 | 100 | 5916 | 5571 | 5251 | 4988 | 4892 | 4811 | 4704 | 4462 | 3658 | 3134 | 2976 | 2841 | 2697 | 2575 | 2458 | 2373 | 2317 | 2165 | 1554 | 1306 | 1300 | 1040 | 0 |
| 9000 | 300 | 6366 | 5916 | 5527 | 5300 | 5260 | 5239 | 5236 | 5194 | 4944 | 4270 | 3692 | 3478 | 3344 | 3257 | 3065 | 2894 | 2713 | 2334 | 1672 | 1380 | 1197 | 899 | 0 |
| 9000 | 500 | 6641 | 6100 | 5668 | 5412 | 5332 | 5205 | 5147 | 5114 | 4925 | 4243 | 3625 | 3364 | 3160 | 2999 | 2887 | 2741 | 2351 | 1983 | 1583 | 1282 | 904 | 626 | 0 |
| 9000 | 1000 | 7221 | 6432 | 5947 | 5625 | 5398 | 4936 | 4705 | 4703 | 4697 | 4204 | 3520 | 3118 | 2854 | 2661 | 2545 | 2118 | 1553 | 1143 | 984 | 729 | 346 | 311 | 0 |
| 9000 | 1500 | 7976 | 7023 | 6356 | 5856 | 5535 | 4958 | 4693 | 4467 | 4189 | 3730 | 3227 | 2850 | 2712 | 2495 | 2034 | 1429 | 1081 | 892 | 546 | 350 | 131 | 119 | 0 |
| 9000 | 2000 | 9111 | 8171 | 7160 | 6258 | 5863 | 5299 | 4835 | 4196 | 3569 | 3097 | 2746 | 2448 | 2312 | 1931 | 1428 | 1027 | 745 | 603 | 345 | 186 | 97 | 48 | 0 |
| 9000 | 2500 | 10108 | 9252 | 8185 | 7162 | 6749 | 6025 | 5004 | 4119 | 3339 | 2797 | 2521 | 2241 | 1942 | 1373 | 1028 | 809 | 570 | 348 | 211 | 136 | 96 | 48 | 0 |
| 9000 | 3000 | 10887 | 10041 | 8998 | 7755 | 7210 | 6399 | 5097 | 4045 | 3240 | 2604 | 2289 | 2038 | 1724 | 1354 | 967 | 708 | 508 | 393 | 282 | 169 | 103 | 49 | 0 |
| 9000 | 3500 | 11622 | 10683 | 9660 | 8227 | 7395 | 6453 | 5140 | 3995 | 3155 | 2487 | 2124 | 1896 | 1643 | 1276 | 952 | 650 | 457 | 419 | 305 | 184 | 107 | 54 | 0 |
| 9000 | 4000 | 12336 | 11283 | 10200 | 8756 | 7669 | 6674 | 5266 | 3983 | 3066 | 2349 | 1931 | 1750 | 1604 | 1375 | 982 | 664 | 558 | 499 | 331 | 190 | 108 | 55 | 0 |
| 9000 | 4500 | 13022 | 11857 | 10680 | 9235 | 7924 | 6936 | 5469 | 4047 | 3026 | 2245 | 1827 | 1719 | 1631 | 1385 | 1037 | 771 | 670 | 615 | 381 | 205 | 109 | 56 | 0 |
| 9000 | 5000 | 13683 | 12418 | 11127 | 9685 | 8259 | 7305 | 5645 | 4183 | 3080 | 2249 | 1836 | 1755 | 1663 | 1416 | 1152 | 908 | 816 | 717 | 445 | 239 | 116 | 57 | 0 |
| 9000 | 5500 | 14332 | 12927 | 11580 | 10180 | 8872 | 7818 | 5764 | 4272 | 3210 | 2355 | 1963 | 1856 | 1760 | 1542 | 1290 | 1071 | 935 | 801 | 512 | 281 | 130 | 62 | 0 |
| 9000 | 6000 | 14961 | 13402 | 11998 | 10758 | 9759 | 8777 | 6149 | 4331 | 3303 | 2421 | 2126 | 2045 | 1937 | 1699 | 1500 | 1219 | 1072 | 892 | 578 | 329 | 157 | 73 | 0 |
| 9000 | 6500 | 15570 | 13909 | 12387 | 11336 | 10230 | 9257 | 6353 | 4416 | 3344 | 2492 | 2346 | 2317 | 2197 | 1985 | 1710 | 1431 | 1170 | 977 | 646 | 378 | 186 | 88 | 0 |
| 9000 | 7000 | 16165 | 14403 | 12779 | 11578 | 10702 | 9737 | 6355 | 4542 | 3482 | 2788 | 2527 | 2402 | 2245 | 2070 | 1785 | 1497 | 1258 | 1058 | 712 | 426 | 216 | 103 | 0 |
| 9000 | 7500 | 16748 | 14881 | 13166 | 11824 | 10747 | 9739 | 6394 | 4637 | 3660 | 3076 | 2877 | 2745 | 2567 | 2142 | 1800 | 1557 | 1339 | 1140 | 778 | 475 | 248 | 118 | 0 |
| 9000 | 8000 | 17323 | 15340 | 13551 | 12107 | 10954 | 9741 | 6556 | 4714 | 3838 | 3419 | 3342 | 3299 | 3162 | 2736 | 2042 | 1620 | 1420 | 1218 | 842 | 525 | 281 | 134 | 0 |
| 10000 | 0 | 4568 | 4337 | 4103 | 3874 | 3760 | 3658 | 3534 | 3274 | 2464 | 1922 | 1596 | 1276 | 1163 | 991 | 849 | 748 | 680 | 625 | 562 | 474 | 410 | 282 | 0 |
| 10000 | 50 | 5126 | 4857 | 4591 | 4336 | 4230 | 4146 | 4040 | 3814 | 3002 | 2482 | 2256 | 2057 | 1890 | 1750 | 1641 | 1555 | 1482 | 1415 | 1139 | 965 | 886 | 645 | 0 |
| 10000 | 100 | 5597 | 5296 | 5007 | 4734 | 4637 | 4565 | 4472 | 4253 | 3512 | 3015 | 2865 | 2728 | 2582 | 2457 | 2347 | 2266 | 2213 | 2105 | 1551 | 1296 | 1211 | 897 | 0 |
| 10000 | 300 | 6039 | 5642 | 5266 | 4961 | 4891 | 4870 | 4868 | 4832 | 4636 | 4016 | 3506 | 3393 | 3315 | 3233 | 2987 | 2781 | 2593 | 2240 | 1383 | 1076 | 1021 | 674 | 0 |
| 10000 | 500 | 6318 | 5831 | 5417 | 5074 | 4965 | 4855 | 4800 | 4765 | 4599 | 4013 | 3353 | 3165 | 3038 | 2871 | 2644 | 2239 | 2002 | 1651 | 1137 | 848 | 796 | 522 | 0 |
| 10000 | 1000 | 6912 | 6184 | 5732 | 5328 | 5073 | 4676 | 4463 | 4376 | 4328 | 3822 | 3225 | 2936 | 2680 | 2484 | 2171 | 1342 | 1033 | 802 | 636 | 460 | 298 | 250 | 0 |
| 10000 | 1500 | 7650 | 6747 | 6118 | 5570 | 5212 | 4712 | 4476 | 4178 | 3843 | 3402 | 2958 | 2559 | 2393 | 2102 | 1655 | 938 | 647 | 547 | 451 | 253 | 127 | 86 | 0 |
| 10000 | 2000 | 8720 | 7826 | 6896 | 6030 | 5591 | 5000 | 4575 | 3974 | 3349 | 2872 | 2468 | 2106 | 1912 | 1402 | 972 | 677 | 480 | 355 | 303 | 155 | 95 | 46 | 0 |
| 10000 | 2500 | 9664 | 8829 | 7895 | 7010 | 6589 | 5726 | 4678 | 3901 | 3191 | 2623 | 2258 | 1901 | 1601 | 1035 | 666 | 533 | 399 | 280 | 195 | 126 | 95 | 47 | 0 |
| 10000 | 3000 | 10400 | 9580 | 8648 | 7587 | 7077 | 6149 | 4779 | 3891 | 3089 | 2444 | 2030 | 1701 | 1419 | 1001 | 647 | 478 | 388 | 300 | 256 | 147 | 97 | 48 | 0 |
| 10000 | 3500 | 11099 | 10200 | 9259 | 8008 | 7253 | 6254 | 4797 | 3830 | 3005 | 2327 | 1877 | 1591 | 1361 | 1062 | 723 | 525 | 414 | 364 | 274 | 151 | 99 | 51 | 0 |
| 10000 | 4000 | 11783 | 10775 | 9774 | 8533 | 7582 | 6503 | 4841 | 3728 | 2909 | 2178 | 1686 | 1470 | 1345 | 1147 | 870 | 641 | 492 | 429 | 291 | 160 | 101 | 53 | 0 |
| 10000 | 4500 | 12444 | 11327 | 10230 | 8989 | 7824 | 6787 | 4940 | 3754 | 2892 | 2087 | 1605 | 1476 | 1410 | 1262 | 1016 | 768 | 590 | 545 | 359 | 188 | 105 | 54 | 0 |
| 10000 | 5000 | 13081 | 11867 | 10655 | 9402 | 8114 | 7092 | 5116 | 3898 | 2977 | 2088 | 1612 | 1562 | 1539 | 1383 | 1130 | 876 | 686 | 639 | 430 | 229 | 112 | 56 | 0 |
| 10000 | 5500 | 13694 | 12355 | 11078 | 9834 | 8616 | 7476 | 5265 | 4034 | 3114 | 2208 | 1742 | 1692 | 1663 | 1496 | 1261 | 993 | 796 | 703 | 490 | 274 | 126 | 59 | 0 |
| 10000 | 6000 | 14268 | 12772 | 11467 | 10320 | 9342 | 8224 | 5687 | 4147 | 3227 | 2286 | 1927 | 1902 | 1821 | 1641 | 1440 | 1183 | 993 | 842 | 553 | 317 | 150 | 70 | 0 |
| 10000 | 6500 | 14835 | 13236 | 11837 | 10808 | 9713 | 8600 | 5912 | 4241 | 3275 | 2378 | 2206 | 2200 | 2098 | 1895 | 1687 | 1382 | 1133 | 942 | 620 | 363 | 179 | 85 | 0 |
| 10000 | 7000 | 15399 | 13708 | 12204 | 11028 | 10083 | 8975 | 5926 | 4267 | 3390 | 2707 | 2422 | 2309 | 2182 | 2003 | 1720 | 1432 | 1212 | 1021 | 686 | 410 | 208 | 99 | 0 |
| 10000 | 7500 | 15952 | 14166 | 12567 | 11253 | 10103 | 8977 | 5940 | 4279 | 3559 | 2995 | 2729 | 2594 | 2483 | 2092 | 1733 | 1484 | 1287 | 1097 | 750 | 459 | 240 | 114 | 0 |
| 10000 | 8000 | 16496 | 14588 | 12938 | 11536 | 10351 | 8979 | 5955 | 4484 | 3783 | 3325 | 3120 | 2988 | 2864 | 2621 | 2015 | 1596 | 1393 | 1182 | 812 | 507 | 272 | 129 | 0 |
| 11000 | 0 | 4294 | 4095 | 3891 | 3694 | 3599 | 3513 | 3407 | 3175 | 2437 | 1880 | 1560 | 1270 | 1146 | 989 | 854 | 757 | 693 | 643 | 565 | 493 | 389 | 267 | 0 |
| 11000 | 50 | 4821 | 4588 | 4356 | 4124 | 4017 | 3938 | 3844 | 3646 | 2917 | 2416 | 2184 | 1985 | 1820 | 1673 | 1579 | 1510 | 1451 | 1375 | 1101 | 938 | 837 | 565 | 0 |
| 11000 | 100 | 5267 | 5006 | 4752 | 4492 | 4379 | 4303 | 4217 | 4027 | 3358 | 2892 | 2727 | 2589 | 2441 | 2299 | 2230 | 2182 | 2146 | 2048 | 1471 | 1039 | 960 | 741 | 0 |
| 11000 | 300 | 5694 | 5346 | 5002 | 4672 | 4560 | 4526 | 4505 | 4452 | 4236 | 3709 | 3392 | 3301 | 3207 | 2996 | 2872 | 2516 | 2259 | 1687 | 1316 | 963 | 652 | 548 | 0 |
| 11000 | 500 | 5966 | 5534 | 5145 | 4765 | 4640 | 4516 | 4468 | 4393 | 4218 | 3706 | 3250 | 3083 | 2957 | 2761 | 2395 | 1926 | 1663 | 1311 | 959 | 728 | 550 | 344 | 0 |
| 11000 | 1000 | 6553 | 5906 | 5430 | 4911 | 4665 | 4365 | 4197 | 4035 | 3837 | 3389 | 2981 | 2693 | 2389 | 2153 | 1682 | 919 | 791 | 696 | 477 | 363 | 245 | 189 | 0 |
| 11000 | 1500 | 7281 | 6484 | 5794 | 5105 | 4739 | 4389 | 4171 | 3806 | 3390 | 2982 | 2564 | 2201 | 1934 | 1567 | 1212 | 675 | 458 | 361 | 295 | 209 | 124 | 65 | 0 |
| 11000 | 2000 | 8285 | 7469 | 6600 | 5732 | 5270 | 4675 | 4221 | 3695 | 3122 | 2619 | 2195 | 1772 | 1484 | 972 | 738 | 554 | 345 | 305 | 228 | 135 | 94 | 46 | 0 |
| 11000 | 2500 | 9166 | 8371 | 7589 | 6807 | 6361 | 5371 | 4332 | 3676 | 3005 | 2461 | 1970 | 1537 | 1265 | 818 | 529 | 399 | 283 | 241 | 192 | 115 | 90 | 47 | 0 |
| 11000 | 3000 | 9861 | 9055 | 8268 | 7343 | 6854 | 5785 | 4446 | 3668 | 2920 | 2257 | 1695 | 1311 | 1109 | 802 | 457 | 322 | 277 | 276 | 237 | 126 | 90 | 48 | 0 |
| 11000 | 3500 | 10518 | 9636 | 8798 | 7680 | 7019 | 5939 | 4491 | 3619 | 2843 | 2127 | 1570 | 1251 | 1101 | 820 | 524 | 407 | 349 | 331 | 247 | 129 | 92 | 49 | 0 |
| 11000 | 4000 | 11153 | 10169 | 9265 | 8151 | 7358 | 6242 | 4605 | 3503 | 2761 | 1952 | 1471 | 1266 | 1160 | 962 | 733 | 572 | 434 | 359 | 274 | 151 | 97 | 50 | 0 |
| 11000 | 4500 | 11765 | 10675 | 9696 | 8592 | 7672 | 6551 | 4757 | 3566 | 2765 | 1999 | 1476 | 1336 | 1256 | 1133 | 922 | 699 | 489 | 408 | 338 | 186 | 104 | 51 | 0 |
| 11000 | 5000 | 12351 | 11158 | 10092 | 8934 | 7863 | 6753 | 4892 | 3707 | 2890 | 2004 | 1540 | 1491 | 1440 | 1299 | 1057 | 804 | 550 | 462 | 397 | 226 | 112 | 53 | 0 |
| 11000 | 5500 | 12922 | 11622 | 10478 | 9284 | 8203 | 7035 | 5020 | 3849 | 2985 | 2108 | 1677 | 1645 | 1578 | 1416 | 1181 | 936 | 664 | 534 | 450 | 272 | 126 | 57 | 0 |
| 11000 | 6000 | 13495 | 12064 | 10871 | 9741 | 8840 | 7663 | 5414 | 4024 | 3128 | 2210 | 1846 | 1823 | 1716 | 1518 | 1326 | 1115 | 920 | 780 | 529 | 307 | 146 | 68 | 0 |
| 11000 | 6500 | 14034 | 12509 | 11220 | 10197 | 9174 | 7996 | 5637 | 4157 | 3262 | 2360 | 2118 | 2112 | 2032 | 1743 | 1521 | 1302 | 1104 | 922 | 602 | 351 | 173 | 82 | 0 |
| 11000 | 7000 | 14562 | 12953 | 11560 | 10403 | 9509 | 8328 | 5655 | 4188 | 3347 | 2669 | 2349 | 2256 | 2110 | 1824 | 1591 | 1396 | 1178 | 994 | 666 | 399 | 203 | 97 | 0 |
| 11000 | 7500 | 15082 | 13381 | 11899 | 10634 | 9563 | 8334 | 5673 | 4209 | 3497 | 2915 | 2635 | 2511 | 2375 | 2020 | 1689 | 1451 | 1248 | 1061 | 729 | 447 | 233 | 111 | 0 |
| 11000 | 8000 | 15594 | 13784 | 12241 | 10923 | 9825 | 8435 | 5692 | 4231 | 3680 | 3152 | 2836 | 2703 | 2634 | 2467 | 1976 | 1595 | 1383 | 1159 | 790 | 493 | 265 | 126 | 0 |

PDO example:

Problem:

Calculate the post-dryout wall temperature for a vertical water-cooled tube of 0.015 m inside diameter.  The following parameters are measured:

Inlet temperature:	250oC

Inlet pressure:		9 500 kPa

Mass flux:		3 961 kg.m-2.s-1

Local pressure:		9 400 kPa

Local heat flux:		1 500 kW.m-2

Local quality:		0.225

Solution:

The following table shows the tabulated heat-transfer coefficients in the 1996 PDO table and the interpolation for the heat-transfer coefficient at conditions of interest.

| Pressure (kPa) | Mass flux (kg.m-2.s-1) | Thermo-dynamic Quality | Heat flux (kW.m-2) | Heat-transfer coefficient (kW.m-2.K-1) | Pressure (kPa) | Mass flux (kg.m-2.s-1) | Thermo-dynamic Quality | Heat flux (kW.m-2) | Heat-transfer coefficient (kW.m-2.K-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9000 | 3000 | 0.2 | 1000 | 3.040 | 10 000 | 3000 | 0.2 | 1000 | 2.877 |
| 9000 | 3000 | 0.2 | 2000 | 2.796 | 10 000 | 3000 | 0.2 | 2000 | 2.094 |
| 9000 | 3000 | 0.4 | 1000 | 5.558 | 10 000 | 3000 | 0.4 | 1000 | 5.760 |
| 9000 | 3000 | 0.4 | 2000 | 4.880 | 10 000 | 3000 | 0.4 | 2000 | 4.486 |
| 9000 | 4000 | 0.2 | 1000 | 5.506 | 10 000 | 4000 | 0.2 | 1000 | 5.250 |
| 9000 | 4000 | 0.2 | 2000 | 4.577 | 10 000 | 4000 | 0.2 | 2000 | 3.985 |
| 9000 | 4000 | 0.4 | 1000 | 8.730 | 10 000 | 4000 | 0.4 | 1000 | 9.058 |
| 9000 | 4000 | 0.4 | 2000 | 7.430 | 10 000 | 4000 | 0.4 | 2000 | 7.405 |
| Interpolation for heat flux of 1500 kW.m-2 |  |  |  |  |  |  |  |  |  |
| 9000 | 3000 | 0.2 | 1500 | 2.918 | 10 000 | 3000 | 0.2 | 1500 | 2.4855 |
| 9000 | 3000 | 0.4 | 1500 | 5.219 | 10 000 | 3000 | 0.4 | 1500 | 5.123 |
| 9000 | 4000 | 0.2 | 1500 | 5.0415 | 10 000 | 4000 | 0.2 | 1500 | 4.6175 |
| 9000 | 4000 | 0.4 | 1500 | 8.08 | 10 000 | 4000 | 0.4 | 1500 | 8.2315 |
| Interpolation for thermodynamic quality of 0.225 |  |  |  |  |  |  |  |  |  |
| 9000 | 3000 | 0.225 | 1500 | 3.2056 | 10 000 | 3000 | 0.225 | 1500 | 2.8152 |
| 9000 | 4000 | 0.225 | 1500 | 5.4213 | 10 000 | 4000 | 0.225 | 1500 | 5.0693 |
| Interpolation for mass flux of 3961 kg.m-2.s-1 |  |  |  |  |  |  |  |  |  |
| 9000 | 3961 | 0.225 | 1500 | 5.3349 | 10 000 | 3961 | 0.225 | 1500 | 4.9814 |
| Interpolation for pressure of 9400 kPa |  |  |  |  |  |  |  |  |  |
| 9400 | 3961 | 0.225 | 1500 | 5.1935 |  |  |  |  |  |

The surface temperature is calculated with

TW=qhPDO+Tbulk=15005.1935+306.7=595.52°C

The 1996 PDO Look-Up Table (PDO-LW-96) of Heat-Transfer Coefficients (in kW.m-2.K-1)

| P = 9000 kPa |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G = 3000 kg.m-2.s-1 |  |  |  |  |  | q (kW.m-2) |  |  |  |  |
| Xth | 50 | 100 | 150 | 200 | 400 | 600 | 800 | 1000 | 2000 | 3000 |
| -0.20 | 2.506 | 2.698 | 2.751 | 2.766 | 2.822 | 1.972 | 1.614 | 1.794 | 2.153 | 2.489 |
| -0.10 | 1.857 | 1.863 | 1.846 | 1.774 | 1.810 | 1.551 | 1.514 | 1.646 | 1.942 | 2.485 |
| -0.05 | 1.306 | 1.338 | 1.330 | 1.335 | 1.339 | 1.449 | 1.413 | 1.451 | 1.935 | 2.512 |
| 0.00 | 1.207 | 1.265 | 1.256 | 1.194 | 1.228 | 1.393 | 1.370 | 1.422 | 1.898 | 2.508 |
| 0.05 | 1.295 | 1.321 | 1.431 | 1.414 | 1.377 | 1.433 | 1.497 | 1.549 | 1.938 | 2.510 |
| 0.10 | 2.570 | 2.404 | 2.536 | 2.543 | 2.259 | 2.200 | 2.133 | 2.149 | 2.204 | 2.521 |
| 0.20 | 5.779 | 5.446 | 4.491 | 4.226 | 3.527 | 3.202 | 3.063 | 3.040 | 2.796 | 2.862 |
| 0.40 | 10.692 | 10.284 | 9.547 | 6.962 | 5.888 | 5.504 | 5.593 | 5.558 | 4.880 | 4.972 |
| 0.60 | 15.656 | 15.285 | 14.887 | 13.952 | 12.078 | 9.591 | 8.997 | 8.309 | 7.497 | 6.315 |
| 0.80 | 19.902 | 19.512 | 19.141 | 18.815 | 16.953 | 14.132 | 12.669 | 11.538 | 10.267 | 8.602 |
| 1.00 | 23.093 | 22.660 | 22.154 | 21.947 | 20.133 | 16.127 | 14.920 | 14.072 | 12.902 | 11.511 |
| 1.20 | 12.822 | 12.733 | 12.643 | 12.534 | 12.209 | 11.279 | 10.700 | 10.841 | 10.815 | 11.364 |

| P = 9000 kPa |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G = 4000 kg.m-2.s-1 |  |  |  |  |  | q (kW.m-2) |  |  |  |  |
| Xth | 50 | 100 | 150 | 200 | 400 | 600 | 800 | 1000 | 2000 | 3000 |
| -0.20 | 2.506 | 2.698 | 2.751 | 2.766 | 2.822 | 1.972 | 1.721 | 1.794 | 2.153 | 2.489 |
| -0.10 | 1.857 | 1.863 | 1.846 | 1.774 | 1.810 | 1.551 | 1.530 | 1.646 | 1.949 | 2.485 |
| -0.05 | 1.346 | 1.448 | 1.605 | 1.587 | 1.497 | 1.449 | 1.504 | 1.494 | 1.935 | 2.512 |
| 0.00 | 1.390 | 1.535 | 1.640 | 1.588 | 1.468 | 1.462 | 1.535 | 1.583 | 1.983 | 2.508 |
| 0.05 | 1.767 | 1.697 | 1.738 | 1.578 | 1.485 | 1.477 | 1.497 | 1.579 | 2.009 | 2.510 |
| 0.10 | 5.310 | 4.963 | 4.770 | 3.517 | 3.057 | 2.658 | 2.493 | 3.028 | 2.940 | 2.887 |
| 0.20 | 8.991 | 8.655 | 8.112 | 7.554 | 6.488 | 5.183 | 4.969 | 5.506 | 4.577 | 4.252 |
| 0.40 | 14.600 | 14.290 | 13.969 | 13.114 | 11.820 | 10.166 | 9.896 | 8.730 | 7.430 | 6.171 |
| 0.60 | 20.105 | 19.773 | 19.459 | 19.163 | 17.659 | 15.418 | 14.191 | 12.658 | 11.022 | 8.988 |
| 0.80 | 25.546 | 25.199 | 24.864 | 24.578 | 23.270 | 21.836 | 20.218 | 18.591 | 15.586 | 12.401 |
| 1.00 | 29.405 | 29.131 | 28.863 | 28.554 | 27.428 | 25.776 | 24.439 | 23.168 | 19.846 | 16.226 |
| 1.20 | 16.430 | 16.370 | 16.312 | 16.263 | 16.068 | 15.904 | 15.708 | 15.620 | 15.285 | 15.152 |

| P = 10000 kPa |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G = 3000 kg.m-2.s-1 |  |  |  |  |  | q (kW.m-2) |  |  |  |  |
| Xth | 50 | 100 | 150 | 200 | 400 | 600 | 800 | 1000 | 2000 | 3000 |
| -0.20 | 2.784 | 3.023 | 3.133 | 3.164 | 3.167 | 2.387 | 2.021 | 2.059 | 2.265 | 2.504 |
| -0.10 | 2.087 | 2.107 | 2.100 | 2.018 | 2.012 | 1.611 | 1.531 | 1.678 | 2.001 | 2.500 |
| -0.05 | 1.456 | 1.356 | 1.334 | 1.337 | 1.374 | 1.486 | 1.378 | 1.547 | 1.904 | 2.498 |
| 0.00 | 1.280 | 1.345 | 1.331 | 1.261 | 1.223 | 1.377 | 1.355 | 1.472 | 1.920 | 2.497 |
| 0.05 | 1.351 | 1.439 | 1.569 | 1.565 | 1.498 | 1.555 | 1.603 | 1.641 | 2.005 | 2.500 |
| 0.10 | 2.791 | 2.642 | 2.801 | 2.886 | 2.501 | 2.398 | 2.289 | 2.265 | 2.144 | 2.530 |
| 0.20 | 6.241 | 5.890 | 4.794 | 4.555 | 3.856 | 3.388 | 3.228 | 2.877 | 2.094 | 2.151 |
| 0.40 | 11.489 | 11.042 | 10.195 | 6.994 | 6.240 | 5.864 | 5.942 | 5.760 | 4.486 | 4.416 |
| 0.60 | 16.734 | 16.363 | 15.985 | 14.966 | 13.171 | 10.997 | 9.875 | 9.521 | 8.232 | 6.872 |
| 0.80 | 21.154 | 20.785 | 20.459 | 20.123 | 18.696 | 16.955 | 15.015 | 13.970 | 11.601 | 8.845 |
| 1.00 | 24.216 | 23.923 | 23.602 | 23.530 | 22.251 | 20.307 | 18.660 | 17.684 | 14.966 | 11.878 |
| 1.20 | 13.535 | 13.456 | 13.390 | 13.256 | 13.021 | 12.681 | 12.461 | 12.402 | 12.045 | 11.935 |
| P = 10000 kPa |  |  |  |  |  |  |  |  |  |  |
| G = 4000 kg.m-2.s-1 |  |  |  |  |  | q (kW.m-2) |  |  |  |  |
| Xth | 50 | 100 | 150 | 200 | 400 | 600 | 800 | 1000 | 2000 | 3000 |
| -0.20 | 2.784 | 3.023 | 3.133 | 3.164 | 3.167 | 2.387 | 2.021 | 2.059 | 2.265 | 2.504 |
| -0.10 | 2.087 | 2.107 | 2.100 | 2.018 | 2.012 | 1.611 | 1.595 | 1.678 | 2.003 | 2.500 |
| -0.05 | 1.456 | 1.438 | 1.688 | 1.640 | 1.531 | 1.497 | 1.567 | 1.551 | 1.953 | 2.498 |
| 0.00 | 1.331 | 1.545 | 1.706 | 1.680 | 1.542 | 1.528 | 1.599 | 1.643 | 2.030 | 2.497 |
| 0.05 | 1.609 | 1.818 | 1.865 | 1.695 | 1.620 | 1.610 | 1.610 | 1.708 | 2.103 | 2.500 |
| 0.10 | 5.894 | 5.523 | 5.309 | 3.789 | 3.286 | 2.826 | 2.710 | 3.124 | 2.950 | 3.127 |
| 0.20 | 9.923 | 9.551 | 8.891 | 8.251 | 7.102 | 5.616 | 5.217 | 5.250 | 3.985 | 3.915 |
| 0.40 | 15.765 | 15.437 | 15.097 | 14.137 | 12.676 | 10.945 | 10.315 | 9.058 | 7.405 | 5.972 |
| 0.60 | 21.553 | 21.201 | 20.867 | 20.554 | 18.945 | 17.018 | 15.373 | 14.091 | 11.827 | 8.968 |
| 0.80 | 27.279 | 26.911 | 26.555 | 26.255 | 24.916 | 23.618 | 22.330 | 20.766 | 16.409 | 12.648 |
| 1.00 | 31.288 | 30.998 | 30.715 | 30.388 | 29.194 | 27.967 | 26.712 | 25.512 | 21.004 | 16.838 |
| 1.20 | 17.442 | 17.360 | 17.279 | 17.210 | 16.933 | 16.701 | 16.499 | 16.382 | 15.835 | 15.407 |

Non-uniform axial power distribution

Problem:

Evaluate the dryout power for a 3.658-m-long tube of 0.0095 m ID, heated with a symmetric-cosine AFD.  The flux profile has been discretized as

| Length (m) | 0.0 | 0.051 | 0.203 | 0.406 | 0.607 | 0.813 | 1.016 | 1.219 | 1.422 | 1.626 | 1.829 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| qloc/qavg | 0.3 | 0.37 | 0.45 | 0.66 | 0.86 | 1.03 | 1.17 | 1.27 | 1.33 | 1.38 | 1.35 |
| Length (m) | 2.032 | 2.235 | 2.438 | 2.642 | 2.845 | 3.048 | 3.251 | 3.454 | 3.607 | 3.658 |  |
| qloc/qavg | 1.38 | 1.33 | 1.27 | 1.17 | 1.03 | 0.86 | 0.66 | 0.45 | 0.37 | 0.30 |  |

The flow conditions are

Pressure:		6 930 kPa

Mass flux:		2 050 kg.m-2.s-1

Inlet subcooling:	280.75 kJ.kg-1

Solution:

To determine the point of incipient dryout, we examine the CHF and AFD curves, in particular we seek the point of tangency, which is a unique intersection.  While curves, which are below dryout, yield no intersection, those above incipient dryout may have two intersections for cosine type AFDs.  We suggest an iteration method based on a binary search.

Since both the dryout location and power are unknown, iteration is required to find the power at which incipient dryout occurs.  The channel is divided into 38 nodes; power values are assumed for a maximum, Nmax, (initially set at 237.1 kW corresponding to 90% dryout quality) and minimum, Nmin, (initially set at 1 kW).  To begin the binary search, the average value, Navg, of 119.0 kW (i.e., ½(237+1)) of the maximum and the minimum powers is taken as an initial guess.

Based on the AFD, the local heat flux and equilibrium quality are calculated at each node (assuming pressure drop is negligible to simplify the calculation).  The CHF value is predicted with the look-up table, and compared against the local heat flux (for quality less than 0) or the corrected local heat flux (for quality greater than or equal to 0).  If the comparison indicates that the heat-flux values coincide at one point, the corresponding power is considered to be the dryout power.  Note that correction for diameter is applied when using the look-up table method.  A heat-flux ratio is calculated

Ratio=Local (or corrected) Heat FluxPredicted CHF Value−1

Incipient dryout is assumed when the maximum ratio obtained for all nodes along the channel (in absolute value) is less than 0.005.  As in a binary search, if the maximum ratio is larger than 0, Nmax is replaced by Navg.  Otherwise, if the maximum ratio is less than 0, Nmin is replaced by Navg.

For the present case, the heat-flux values calculated in the first iteration are shown in Table 1.  Since the maximum ratio is -0.339 (less than 0), Nmin is replaced by Navg (i.e., 119.0 kW).  The new Navg will be ½(237+119) and becomes 176 kW.  Iteration is continued until the unique point is located.  The variation of power and the corresponding maximum heat-flux ratio are presented in Table III.4-2.

After 4 iterations, the corrected local heat flux coincides with the predicted CHF from the look-up table at a location of 3.353 m from the inlet (Table III.4-3).  Therefore, the corresponding power, which is 132.6 kW, is taken as the dryout power.

Table 1: Heat-Flux Values Calculated in the First Iteration

| Node | Distance | Quality | Local Heat Flux | BLA Heat Flux | Predicted CHF | Ratio |
| --- | --- | --- | --- | --- | --- | --- |
|  | (m) | (%) | (kW.m-2) | (kW.m-2) | (kW.m-2) |  |
| 1 | 0.051 | -17.54 | 401.1 | 401.1 | 7181.9 | -0.944 |
| 2 | 0.127 | -17.10 | 444.5 | 444.5 | 7139.1 | -0.937 |
| 3 | 0.203 | -16.62 | 487.9 | 487.9 | 7092.0 | -0.931 |
| 4 | 0.305 | -15.86 | 601.7 | 601.7 | 7018.6 | -0.914 |
| 5 | 0.406 | -14.95 | 715.5 | 715.5 | 6926.1 | -0.896 |
| 6 | 0.508 | -13.88 | 824.0 | 824.0 | 6734.9 | -0.877 |
| 7 | 0.610 | -12.66 | 932.4 | 932.4 | 6516.6 | -0.856 |
| 8 | 0.711 | -11.31 | 1024.5 | 1024.5 | 6273.5 | -0.836 |
| 9 | 0.813 | -9.82 | 1116.7 | 1116.7 | 5991.6 | -0.813 |
| 10 | 0.914 | -8.22 | 1192.6 | 1192.6 | 5557.2 | -0.785 |
| 11 | 1.016 | -6.51 | 1268.5 | 1268.5 | 5094.3 | -0.750 |
| 12 | 1.118 | -4.72 | 1322.7 | 1322.7 | 4667.6 | -0.716 |
| 13 | 1.219 | -2.84 | 1376.9 | 1376.9 | 4568.8 | -0.698 |
| 14 | 1.321 | -9.17 | 1409.4 | 1409.4 | 4466.7 | -0.684 |
| 15 | 1.422 | 1.06 | 1442.0 | 1442.0 | 4348.9 | -0.668 |
| 16 | 1.524 | 3.07 | 1469.1 | 1445.1 | 4286.9 | -0.657 |
| 17 | 1.626 | 5.13 | 1496.2 | 1459.9 | 4176.5 | -0.641 |
| 18 | 1.727 | 7.19 | 1479.9 | 1467.9 | 3861.6 | -0.616 |
| 19 | 1.829 | 9.24 | 1463.7 | 1468.7 | 3575.3 | -0.590 |
| 20 | 1.930 | 11.28 | 1479.9 | 1469.3 | 3423.0 | -0.567 |
| 21 | 2.032 | 13.34 | 1496.2 | 1472.1 | 3291.7 | -0.545 |
| 22 | 2.134 | 15.40 | 1469.1 | 1473.5 | 3093.9 | -0.525 |
| 23 | 2.235 | 17.42 | 1442.0 | 1471.4 | 3003.2 | -0.519 |
| 24 | 2.337 | 19.39 | 1409.4 | 1466.6 | 2908.5 | -0.515 |
| 25 | 2.438 | 21.33 | 1376.9 | 1459.7 | 2778.2 | -0.504 |
| 26 | 2.540 | 23.20 | 1322.7 | 1450.1 | 2596.7 | -0.490 |
| 27 | 2.642 | 25.00 | 1268.5 | 1437.8 | 2428.5 | -0.477 |
| 28 | 2.743 | 26.70 | 1192.6 | 1422.5 | 2173.6 | -0.451 |
| 29 | 2.845 | 28.31 | 1116.7 | 1404.1 | 1942.6 | -0.425 |
| 30 | 2.946 | 29.79 | 1024.5 | 1382.6 | 1706.8 | -0.399 |
| 31 | 3.048 | 31.15 | 932.4 | 1358.2 | 1503.3 | -0.379 |
| 32 | 3.150 | 32.37 | 824.0 | 1330.8 | 1294.2 | -0.363 |
| 33 | 3.251 | 33.43 | 715.5 | 1300.5 | 1102.1 | -0.350 |
| 34 | 3.353 | 34.35 | 601.7 | 1267.7 | 915.3 | -0.342 |
| 35 | 3.454 | 35.10 | 487.9 | 1232.5 | 738.9 | -0.339 |
| 36 | 3.531 | 35.59 | 444.5 | 1205.5 | 673.7 | -0.340 |
| 37 | 3.607 | 36.03 | 401.1 | 1178.8 | 609.5 | -0.341 |
| 38 | 3.658 | 36.32 | 325.2 | 1161.9 | 530.3 | -0.387 |

Table 2: Variation of Power and the Corresponding Maximum Heat-Flux Ratio

Iteration	Power, Navg (kW)	Maximum Heat-Flux Ratio

1			 119.0			-0.340

2			 176.0		 	1.730

3			 147.2			0.510

4			 132.6	 		0.004

Table 3: Heat-Flux Values Calculated in the Last Iteration

Node	Distance	Quality	Local Heat Flux	BLA Heat Flux	Predicted CHF	Ratio

(m)		(%)	(kW.m-2)	(kW.m-2)	(kW.m-2)

1	0.051		-17.51	450.8		450.8		7178.6		-0.937

2	0.127		-17.02	499.6		499.6		7130.6		-0.929

3	0.203		-16.47	548.3		548.3		7077.6		-0.922

4	0.305		-15.62	676.3		676.3		6995.1		-0.903

5	0.406		-14.60	804.2		804.2		6862.6		-0.882

6	0.508		-13.40	926.1		926.1		6647.6		-0.860

7	0.610		-12.03	1047.9		1047.9		6402.3		-0.836

8	0.711		-10.50	1151.5		1151.5		6129.1		-0.812

9	0.813		-8.83	1255.1		1255.1		5723.0		-0.780

10	0.914		-7.03	1340.4		1340.4		5234.8		-0.743

11	1.016		-5.11	1425.7		1425.7		4714.5		-0.697

12	1.118		-3.09	1486.6		1486.6		4581.9		-0.675

13	1.219		-9.94	1547.6		1547.6		4470.8		-0.653

14	1.321		1.17	1584.1		1584.1		4341.3		-0.635

15	1.422		3.40	1620.7		1589.6		4278.2		-0.621

16	1.524		5.66	1651.2		1607.8		4120.1		-0.599

17	1.626		7.98	1681.6		1624.3		3869.2		-0.565

18	1.727		10.30	1663.3		1634.9		3534.7		-0.529

19	1.829		12.59	1645.1		1638.4		3310.4		-0.503

20	1.930		14.88	1663.3		1640.8		3162.4		-0.474

21	2.032		17.20	1681.6		1645.0		3136.9		-0.463

22	2.134		19.52	1651.2		1647.5		3030.9		-0.455

23	2.235		21.78	1620.7		1646.3		2875.7		-0.436

24	2.337		24.01	1584.1		1642.1		2705.5		-0.414

25	2.438		26.18	1547.6		1635.5		2499.6		-0.380

26	2.540		28.28	1486.6		1626.1		2234.8		-0.334

27	2.642		30.30	1425.7		1613.5		1995.9		-0.285

28	2.743		32.22	1340.4		1597.7		1763.5		-0.239

29	2.845		34.02	1255.1		1578.4		1554.5		-0.192

30	2.946		35.69	1151.5		1555.7		1346.1		-0.144

31	3.048		37.22	1047.9		1529.7		1160.6		-0.097

32	3.150		38.59	926.1		1500.5		976.7		-0.051

33	3.251		39.79	804.2		1468.0		813.3		-0.011

34	3.353		40.81	676.3		1432.5		673.0		0.004

35	3.454		41.66	548.3		1394.5		543.5		0.008

36	3.531		42.21	499.6		1365.2		495.6		0.008

37	3.607		42.70	450.8		1336.2		448.4		0.005

38	3.658		43.03	365.5		1317.7		393.7		-0.071

Assignment on CHF and post-dryout heat transfer:

A pressure tube, with inside diameter of 103.86 mm, contains twelve 37-element bundles of the following dimension:

Bundle length: 495.3 mm

Element outer diameter: 13.06 mm

Total power generated over the bundle string is 6.3 MW, and the axial power profile is uniform.  The channel is cooled with a flow of light water at the following conditions:

Outlet pressure: 9 MPa

Mass flow rate: 17 kg/s

Inlet fluid temperature: 265C

Evaluate the following:

- Dryout power ratio for this channel, and

- Surface temperature distribution along the bundle string at an overpower of 10% (overpower is defined as (String-Power/Dryout-Power – 1)*100%)

Previous research findings:

- Heat-transfer coefficient at nucleate boiling for the 37-element bundle is 120 kW/(m2 K) at the pressure of 9 MPa, mass flux of 5 Mg/(m2 s) and quality of 20%.

- Heat-transfer coefficient at single-phase liquid flow is about 50% of that at nucleate boiling.

- CHF in the 37-element bundle is 20% lower than that in tubes based on cross-sectional average flow conditions.

- Heat-transfer coefficient at fully developed film boiling for the 37-element bundle is the same as that for tubes.

- Overall loss coefficient is 1.7 for a 37-element bundle inside a pressure tube of 103.86 mm.

- The homogeneous equation is the most appropriate for two-phase multiplier.

Basic assumptions:

- Nucleate boiling heat transfer coefficient does not vary with flow conditions.

- Fully developed film-boiling heat transfer is reached once local heat flux exceeds CHF.

- Equilibrium flow in the channel.

- Pressure drop due to acceleration is negligible.

More nodes can be introduced to improve the accuracy of the predictions of dryout location and power.  Theoretically, an infinite number of nodes should be used for a smooth profile.  For a stepped profile, the number of nodes corresponds to the number of steps since dryout generally occurs at the end of a step.